SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- ANALYTICAL PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL 1- ANALYTICAL PROBLEM SOLVING |
| Weightage: | 45% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

1. A Booth's multiplication implementation produces an incorrect result when multiplying            -13 *9. Analyze the execution steps to identify the incorrect addition, subtraction, or arithmetic shift operation responsible for the error. Correct the algorithm and suggest an optimization that minimizes the number of execution cycles.

2. A processor executing the restoring division algorithm produces an incorrect quotient for the operation 29 /5. Examine the sequence of iterations to determine the faulty restore step, correct the error, and recommend an optimization to improve the algorithm's efficiency.

3. During the execution of the non-restoring division algorithm, incorrect quotient bits are generated after the fourth iteration. Investigate the source of the logic error, modify the algorithm to eliminate the issue, and verify the corrected quotient and remainder.

4. A Booth's multiplication unit incorrectly performs logical right shifts instead of arithmetic right shifts, leading to erroneous results for signed multiplication. Identify the cause of the error, explain why arithmetic right shifting is required, and demonstrate how it ensures correct signed multiplication.

5. A floating-point addition program based on the IEEE 754 single-precision format produces an incorrect result because the operands' exponents are not aligned before addition. Identify the implementation error, explain the correct exponent alignment procedure, and propose an optimized approach to improve both accuracy and execution efficiency.

Code of Conduct:

I faraj Khaleel P(192525134) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Problem Understanding (20%) | Method Selection (20%) | Solution Process (25%) | Accuracy of Calculation (20%) | Interpretation of Results (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |

Q1. Booth's Multiplication (-13 × 9)

Problem Analysis The Booth's multiplication unit generates an incorrect result while multiplying −13 × 9. Since Booth's algorithm is designed for signed multiplication using two's complement representation, every iteration must correctly perform addition/subtraction and an arithmetic right shift. Any mistake in these operations produces an incorrect final product.

Analysis of the Error The incorrect result occurs because one of the following operations is executed incorrectly: • Wrong addition/subtraction between accumulator and multiplicand. • Arithmetic Right Shift (ARS) replaced with Logical Right Shift (LRS). Logical right shift inserts 0 into the most significant bit, changing the sign of negative numbers. As a result, the multiplication process becomes incorrect.

Correct Solution 1. Load Multiplicand (M = −13) and Multiplier (Q = 9). 2. Check Q₀ and Q₋₁. 3. Perform operations based on bit pairs: • 01 → A = A + M • 10 → A = A − M • 00 or 11 → No operation 4. Perform Arithmetic Right Shift after every iteration. 5. Repeat for all bits. Expected Result: −13 × 9 = −117

Optimization Modified Booth's Algorithm (Radix-4) reduces the number of addition and subtraction operations, decreasing execution cycles and improving processor performance.

Final Result After correcting the subtraction operation and replacing logical shift with arithmetic shift, the multiplication result becomes −117, which is the correct signed product.

Q2. Restoring Division (29 ÷ 5)

Problem Analysis The processor generates an incorrect quotient while performing 29 ÷ 5 using the Restoring Division Algorithm. The error occurs because the restore operation is skipped after obtaining a negative remainder.

Analysis of the Error During each iteration: • Shift A and Q left. • Subtract divisor from accumulator. • If accumulator becomes negative, the divisor must be added back. The implementation failed to restore the accumulator after a negative subtraction, causing all remaining quotient bits to become incorrect.

Correct Solution 1. Load Dividend = 29 and Divisor = 5. 2. Shift accumulator and quotient left. 3. Perform subtraction. 4. If result is negative: Restore accumulator and set quotient bit = 0. 5. Otherwise: Keep subtraction result and set quotient bit = 1. 6. Repeat until all bits are processed. Expected Output: Quotient = 5, Remainder = 4

Optimization The Non-Restoring Division Algorithm removes unnecessary restore operations, reducing execution time and improving efficiency.

Final Result The corrected algorithm produces the correct quotient 5 and remainder 4.

Q3. Non-Restoring Division

Problem Analysis The Non-Restoring Division Algorithm generates incorrect quotient bits after the fourth iteration because the wrong arithmetic operation is selected based on the accumulator's sign.

Analysis of the Error The algorithm should perform subtraction when the accumulator is positive and addition when the accumulator is negative. Instead, subtraction was performed even when the accumulator was negative, producing incorrect quotient bits.

Correct Solution 1. Shift accumulator and quotient left. 2. If accumulator is positive: Subtract divisor. 3. If accumulator is negative: Add divisor. 4. Set quotient bit (Positive accumulator → 1, Negative accumulator → 0). 5. After the final iteration, restore the accumulator if necessary. Expected Output: Quotient = 5, Remainder = 4

Optimization Hardware sign detection allows automatic selection of addition or subtraction, reducing execution delay.

Final Result The corrected algorithm generates the correct quotient and remainder with accurate quotient bits.

Q4. Booth's Multiplication Using Arithmetic Right Shift

Problem Analysis The Booth's multiplication hardware incorrectly performs Logical Right Shift instead of Arithmetic Right Shift, causing incorrect signed multiplication.

Analysis of the Error • Logical Right Shift: Inserts 0 into the MSB, changes the sign of negative numbers, and produces incorrect multiplication. • Arithmetic Right Shift: Replicates the sign bit, preserves two's complement representation, and produces correct signed multiplication. Example: • Before Shift: 11110011 • Arithmetic Right Shift: 11111001 • Logical Right Shift: 01111001 The logical shift changes the number from negative to positive, leading to incorrect results.

Correct Solution Replace every Logical Right Shift with an Arithmetic Right Shift during Booth's multiplication.

Optimization Using an Arithmetic Shift Register in hardware eliminates additional correction steps and improves processor performance.

Final Result Arithmetic Right Shift preserves signed values and ensures correct Booth's multiplication.

Q5. IEEE 754 Floating-Point Addition

Problem Analysis The floating-point addition program produces an incorrect result because the operands' exponents are not aligned before addition.

Analysis of the Error In IEEE 754 arithmetic, mantissas cannot be added directly when exponents differ. For instance, if Operand A has Exponent = 130 and Operand B has Exponent = 127, the mantissa of Operand B must first be shifted right by three positions. Without exponent alignment, the computed result becomes incorrect.

Correct Solution 1. Compare exponents. 2. Shift the mantissa of the smaller exponent. 3. Make both exponents equal. 4. Add mantissas. 5. Normalize the result, round the value, and store the final floating-point number.

Optimization Hardware barrel shifters and parallel exponent comparison significantly reduce execution time while improving numerical accuracy.

Final Result After proper exponent alignment, the floating-point addition follows the IEEE 754 standard and produces the correct result with improved accuracy and execution efficiency.