Department of Computer Science and Engineering

CSA1203 – Computer Architecture

Design and Analysis of an Energy-Efficient Processor and Hierarchical Memory System

ASSIGNMENT REPORT

| FIELD | DETAILS |
| --- | --- |
| COURSE CODE | CSA1203 |
| COURSE NAME | COMPUTER ARCHITECTURE |
| ASSIGNMENT TITLE | ENERGY-EFFICIENT PROCESSOR & MEMORY SYSTEM DESIGN |
| STUDENT NAME | FARAJ KHALEEL P |
| REGISTER NUMBER | 192525134 |
| ACADEMIC YEAR | 2025–2029 |

# SECTION 1: FUNCTIONAL UNITS, INSTRUCTION EXECUTION, ADDRESSING MODES & PERFORMANCE ANALYSIS

CO Mapping: CO1

## 1.1 PROBLEM CONTEXT — INDUSTRIAL MONITORING PROCESSOR

- A smart manufacturing facility requires a real-time industrial monitoring processor (IMP-1) that collects data from multiple distributed sensors (temperature, pressure, vibration, flow), processes it with deterministic latency, and triggers actuators or alerts. The processor must operate under strict power budgets (≤ 5W), maintain real-time guarantees (< 1 ms response), and support 24/7 continuous operation.

- The design uses a 32-bit RISC architecture inspired by MIPS, clocked at 500 MHz, with a 5-stage pipeline and dedicated sensor I/O interfaces.

## 1.2 FUNCTIONAL UNITS OF THE IMP-1 PROCESSOR

### 1.2.1 Central Processing Unit (CPU)

- The CPU is the core computation engine comprising the following sub-units:

- Arithmetic Logic Unit (ALU): Performs 32-bit integer arithmetic (ADD, SUB, MUL), logical operations (AND, OR, XOR, NOT), and comparisons (SLT, SLTU). Single-cycle for all integer ops at 500 MHz.

- Control Unit (CU): Decodes the 6-bit opcode and 6-bit function code, generating control signals for MUX selectors, register write-enable, ALU operation, memory read/write, and branch/jump.

- Register File: 32 general-purpose 32-bit registers (R0–R31). R0 is hardwired to 0. Dual read ports and one write port support simultaneous decode and writeback.

- Program Counter (PC): 32-bit register updated each cycle. Supports sequential increment (PC+4), branch (PC + offset×4), and jump (J-type absolute).

- Instruction Register (IR): Holds the fetched 32-bit instruction during the decode stage.

### 1.2.2 Memory Subsystem

A Harvard-inspired split-cache architecture separates instruction and data paths to eliminate structural hazards:

- L1 Instruction Cache (L1-I): 32 KB, 4-way set-associative, 2-cycle hit latency. Feeds the IF stage.

- L1 Data Cache (L1-D): 32 KB, 4-way set-associative, 2-cycle hit latency. Serves MEM stage load/store operations.

- L2 Unified Cache: 256 KB, 8-way, 8-cycle hit latency. Shared between instruction and data miss streams.

- L3 Shared Cache: 2 MB, 16-way, 30-cycle hit latency. Last on-chip level before off-chip DRAM.

- Main Memory (LPDDR4 DRAM): 2 GB, 200-cycle latency, 51.2 GB/s peak bandwidth for burst sensor log transfers.

### 1.2.3 Bus Architecture

| BUS | WIDTH | ROLE | PROTOCOL |
| --- | --- | --- | --- |
| ADDRESS BUS | 32-BIT | CARRY MEMORY/IO ADDRESSES | SYNCHRONOUS, ONE-CYCLE VALID |
| DATA BUS | 32-BIT | TRANSFER INSTRUCTIONS AND DATA | SPLIT-TRANSACTION WITH HANDSHAKE |
| CONTROL BUS | 8-BIT | R/W, INTERRUPT, CLOCK, RESET SIGNALS | ACTIVE-LOW, LEVEL-TRIGGERED |
| SENSOR I/O BUS | 16-BIT SPI | ADC SENSOR DATA INGESTION | UP TO 10 MHZ SPI, INTERRUPT-DRIVEN |

### 1.2.4 I/O Components

- Analog-to-Digital Converter (ADC) Interface: 12-bit resolution, 8 multiplexed sensor channels. Interrupt-driven sampling at up to 1 MSPS. Connected via the 16-bit SPI sensor bus.

- UART Controller: 115200 baud RS-232 for diagnostic output and PC communication.

- Timer/Counter Unit: Two 32-bit hardware timers for periodic sensor polling and watchdog functionality.

- DMA Controller: 4-channel DMA for autonomous sensor-buffer transfers, freeing the CPU for computation.

- GPIO: 32 programmable pins for digital sensor/actuator interfacing.

## 1.3 Instruction Execution Cycle

The IMP-1 uses the classic 5-stage pipeline. The table below traces a sensor LOAD instruction (LW R5, 0x10(R3)) through each stage:

| STAGE | OPERATION | ACTIVE UNITS | RESULT |
| --- | --- | --- | --- |
| IF – INSTRUCTION FETCH | PC → L1-I CACHE; FETCH 32-BIT INSTRUCTION WORD | PC, L1-I CACHE, IR | IR ← 0X8C650010 (LW ENCODING) |
| ID – INSTRUCTION DECODE | DECODE OPCODE (LW); READ BASE REGISTER R3 FROM REGISTER FILE | CONTROL UNIT, REGISTER FILE | CU SIGNALS: MEMREAD=1, REGWRITE=1; R3 VALUE = 0X2000 |
| EX – EXECUTE | ALU COMPUTES EFFECTIVE ADDRESS: R3 + 0X10 = 0X2010 | ALU, SIGN-EXTENDER | EX/MEM.ALUOUT = 0X00002010 |
| MEM – MEMORY ACCESS | SEND ADDRESS 0X2010 TO L1-D CACHE; READ 32-BIT WORD | L1-D CACHE, DATA BUS | MEM/WB.LMD = SENSOR_VALUE |
| WB – WRITE BACK | WRITE SENSOR_VALUE TO R5 IN REGISTER FILE | REGISTER FILE | R5 ← SENSOR_VALUE |

## 1.4 Addressing Modes

The IMP-1 supports six addressing modes, each suited to specific sensor-processing patterns:

| MODE | SYNTAX | EFFECTIVE ADDRESS / VALUE | USE CASE IN IMP-1 |
| --- | --- | --- | --- |
| IMMEDIATE | ADDI R1, R0, 100 | OPERAND = 100 (CONSTANT IN INSTRUCTION) | INITIALIZE THRESHOLD CONSTANTS FOR SENSOR ALARMS |
| REGISTER | ADD R3, R1, R2 | EA = REGISTER[R1] + REGISTER[R2] | ACCUMULATE SENSOR READINGS IN REGISTERS |
| DIRECT (ABSOLUTE) | LW R4, 0X4000 | EA = 0X4000 | ACCESS FIXED SENSOR MEMORY-MAPPED I/O REGISTER |
| REGISTER INDIRECT | LW R5, 0(R6) | EA = R6 | TRAVERSE SENSOR DATA ARRAYS VIA POINTER |
| BASE + OFFSET | LW R7, 8(R8) | EA = R8 + 8 | ACCESS FIELDS WITHIN A SENSOR DATA STRUCT |
| PC-RELATIVE | BEQ R1, R2, LABEL | EA = PC + 4 + OFFSET×4 | CONDITIONAL BRANCH TO ALARM-HANDLER ROUTINE |

## 1.5 PERFORMANCE ANALYSIS — CPI, CLOCK CYCLES & EXECUTION TIME

### 1.5.1 Workload Definition

- Benchmark workload: one iteration of the sensor monitoring loop processing 10,000 sensor readings. The loop performs: (1) read sensor via LW, (2) compare against threshold, (3) compute moving average, (4) conditional branch if alarm, (5) store result.

| INSTRUCTION TYPE | COUNT | PROPORTION | INDIVIDUAL CPI (PIPELINED) |
| --- | --- | --- | --- |
| ALU / ARITHMETIC (R-TYPE) | 35,000 | 35% | 1.0 |
| LOAD / STORE (MEMORY) | 25,000 | 25% | 1.0 (HIT) / 3.0 (MISS) |
| BRANCH / JUMP | 20,000 | 20% | 1.0 (PREDICTED) / 3.0 (MISPREDICTED) |
| I/O READ (SENSOR POLL) | 15,000 | 15% | 2.0 |
| MULTIPLY / DIVIDE | 5,000 | 5% | 4.0 |
| TOTAL | 100,000 | 100% | — |

### 1.5.2 CPI Calculation:-

Weighted CPI = Σ (Proportion × CPI_i)

Assumptions: L1-D hit rate = 92%; branch prediction accuracy = 88%; load/store miss penalty = 2 extra cycles; misprediction penalty = 2 extra cycles.

CPI_load = 0.92 × 1.0 + 0.08 × 3.0 = 0.92 + 0.24 = 1.16

CPI_branch = 0.88 × 1.0 + 0.12 × 3.0 = 0.88 + 0.36 = 1.24

Weighted CPI = (0.35 × 1.0) + (0.25 × 1.16) + (0.20 × 1.24) + (0.15 × 2.0) + (0.05 × 4.0)

= 0.350 + 0.290 + 0.248 + 0.300 + 0.200 = 1.388

### 1.5.3 Execution Time:-

Clock Period T = 1/f = 1/500×10⁶ = 2 ns

Total Clock Cycles = IC × CPI = 100,000 × 1.388 = 138,800 cycles

Execution Time = 138,800 × 2 ns = 277.6 µs per 10,000 sensor readings

Throughput = 100,000 / 277.6×10⁻⁶ = 360.3 MIPS

1.5.4 PERFORMANCE SUMMARY TABLE

| METRIC | UNPIPELINED BASELINE | PIPELINED IMP-1 | IMPROVEMENT |
| --- | --- | --- | --- |
| CLOCK FREQUENCY | 500 MHZ | 500 MHZ | — |
| CPI | 4.00 (SEQUENTIAL) | 1.388 | 2.88× BETTER |
| TOTAL CYCLES (10K READINGS) | 400,000 | 138,800 | 65.3% FEWER CYCLES |
| EXECUTION TIME | 800 µS | 277.6 µS | 2.88× FASTER |
| THROUGHPUT (MIPS) | 125 | 360.3 | 2.88× HIGHER |
| POWER ESTIMATE | ~8W | ~5W | 37.5% REDUCTION |

Table 1.1: Performance comparison — unpipelined vs 5-stage pipelined IMP-1 processor.

# SECTION 2: PIPELINED PROCESSOR ORGANIZATION & HAZARD ANALYSIS

CO MAPPING: CO3  |  MARKS: 30

## 2.1 5-STAGE PIPELINE DATAPATH DESIGN

The IMP-1 implements a classic 5-stage in-order pipeline: Instruction Fetch (IF), Instruction Decode / Register Read (ID), Execute (EX), Memory Access (MEM), Write Back (WB). Pipeline registers (IF/ID, ID/EX, EX/MEM, MEM/WB) latch intermediate values between stages, enabling four to five instructions to overlap execution.

Key datapath components:

- Forwarding Unit: Detects EX→EX and MEM→EX data dependencies; routes ALU results from EX/MEM or MEM/WB registers directly to the ALU input MUX, bypassing the register file read.

- Hazard Detection Unit: Sits in the ID stage; detects load-use hazards (where a LOAD in EX produces a value needed by the next instruction in ID); inserts one pipeline stall (bubble) and freezes IF/ID and PC.

- Branch Prediction Unit: 2-bit saturating counter predictor with 512-entry Branch History Table (BHT). Achieves 88% accuracy on the sensor-monitoring loop conditional branches.

- Control Path: Single CU in ID; control signals propagate forward through pipeline registers (ID/EX, EX/MEM, MEM/WB) alongside data.

### 2.1.1 Pipeline Timing Diagram (6 Instructions)

The diagram below shows 6 instructions executing concurrently after the pipeline fills (steady state):

| Instruction | Cycle 1 | Cycle 2 | Cycle 3 | Cycle 4 | Cycle 5 | Cycle 6 | Cycle 7 | Cycle 8 | Cycle 9 | Cycle 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I1: LW R1,0(R2) | IF | ID | EX | MEM | WB |  |  |  |  |  |
| I2: ADD R3,R1,R4 |  | IF | ID | **stall** | EX | MEM | WB |  |  |  |
| I3: SW R3,4(R2) |  |  | IF | **stall** | ID | EX | MEM | WB |  |  |
| I4: BEQ R3,R5,L |  |  |  |  | IF | ID | EX | MEM | WB |  |
| I5: ADDI R6,R0,1 |  |  |  |  |  | IF | ID | EX | MEM | WB |
| I6: LW R7,8(R2) |  |  |  |  |  |  | IF | ID | EX | MEM |

Note: I2 stalls one cycle due to a load-use hazard (I1 loads R1, I2 reads R1 immediately). Forwarding paths handle all other data dependencies without stalls.

## 2.2 HAZARD ANALYSIS

### 2.2.1 Data Hazards (Read-After-Write — RAW)

- A RAW hazard occurs when an instruction reads a register before a preceding instruction has written its result. Example from the sensor averaging code:

| INSTRUCTION | STAGE CONFLICT | DEPENDENCY | RESOLUTION |
| --- | --- | --- | --- |
| LW R1, 0(R2) ← READS SENSOR | MEM→WB WRITES R1 | I+1 NEEDS R1 IN EX | 1-CYCLE STALL (LOAD-USE; NO FORWARDING PATH FROM MEM/WB TO EX INPUT BEFORE WB) |
| ADD R3, R1, R4 ← USE R1 | EX READS R1 FROM ID | RAW ON R1 | STALL INSERTED; THEN FORWARD MEM/WB.LMD → ALU INPUT A |
| SUB R5, R3, R6 | EX READS R3 PRODUCED IN EX OF ADD | RAW ON R3 | EX/MEM FORWARDING: ROUTE ADD'S ALU RESULT → SUB'S ALU INPUT A. ZERO STALLS. |
| MUL R7, R5, R8 | EX READS R5 PRODUCED IN MEM OF SUB | RAW ON R5 | MEM/WB FORWARDING: ROUTE SUB RESULT → MUL'S ALU INPUT A. ZERO STALLS. |

Impact without forwarding: 2 stall cycles per RAW pair → CPI increase of 0.35 × 2 × 0.15 ≈ 0.10 added CPI. Forwarding eliminates ~90% of RAW stalls, recovering nearly all this overhead.

### 2.2.2 Control Hazards (Branch Hazards)

- A control hazard occurs when a branch instruction in the EX stage has already allowed two subsequent instructions into IF and ID — those instructions may be from the wrong execution path.

- Resolution strategy for IMP-1:

- 2-bit Saturating Counter Predictor: Predict branch outcome using a Branch History Table (BHT) with 512 entries. States: Strongly Not-Taken (00) → Weakly Not-Taken (01) → Weakly Taken (10) → Strongly Taken (11). For the sensor loop's back-edge branch (repeating), the predictor quickly saturates to Strongly Taken.

- On misprediction: flush the 2 incorrectly-fetched instructions (convert to NOPs / bubbles) and redirect PC to the correct target. Penalty = 2 cycles.

- Branch penalty contribution to CPI: 0.20 × 0.12 misprediction_rate × 2 cycles = 0.048 added CPI (after prediction).

### 2.2.3 STRUCTURAL HAZARDS

- A structural hazard occurs when two pipeline stages simultaneously require the same hardware resource. In a unified-memory design, the IF stage (fetching next instruction) and MEM stage (load/store) would both need memory access in the same cycle.

| STRUCTURAL HAZARD SCENARIO | STAGE CONFLICT | RESOLUTION IN IMP-1 |
| --- | --- | --- |
| SINGLE MEMORY UNIT: IF AND MEM BOTH ACCESS MEMORY IN CYCLE 5 | IF VS MEM → MEMORY CONFLICT | HARVARD SPLIT-CACHE: SEPARATE L1-I (INSTRUCTION) AND L1-D (DATA) CACHES — ZERO STRUCTURAL HAZARD |
| SINGLE WRITE PORT TO REGISTER FILE: WB AND DECODE BOTH NEED REGISTER FILE | WB WRITES, ID READS SAME REGISTER | DUAL-PORT REGISTER FILE WITH WRITE-IN-FIRST-HALF / READ-IN-SECOND-HALF OF CYCLE; FORWARDING HANDLES REMAINING CONFLICT |
| MULTI-CYCLE ALU (MUL TAKES 4 CYCLES) BLOCKS EX STAGE | SUBSEQUENT INSTRUCTIONS STALL AT ID | NON-BLOCKING EXECUTION UNIT: MUL RUNS IN A SEPARATE 4-STAGE MULTIPLY PIPELINE; INT ALU REMAINS FREE FOR SUBSEQUENT INSTRUCTIONS |

## 2.3 Hazard Mitigation — Summary of Techniques

| Hazard Type | Technique Applied | Cycles Saved per Occurrence | HW Cost |
| --- | --- | --- | --- |
| RAW (general ALU→ALU) | EX/MEM forwarding | 2 cycles (eliminates stall) | Forwarding MUX + detection logic |
| RAW (load-use) | MEM/WB forwarding + 1-cycle stall | 1 cycle (saves 1 of 2) | Hazard detect unit + stall FSM |
| WAW / WAR | In-order pipeline inherently prevents | N/A | None |
| Control (branch) | 2-bit BHT predictor | 1.76 cycles avg (88% hit rate) | 512-entry BHT table (2 kB) |
| Structural (memory) | Split L1-I / L1-D caches | 1 cycle per overlap | 2× 32 KB L1 SRAM arrays |
| Structural (ALU) | Separate multiply pipeline | Variable | Dedicated 4-stage MUL unit |

## 2.4 Superscalar, Out-of-Order Execution & Speculative Execution

### 2.4.1 Technique Comparison

| Technique | Mechanism | IPC Potential | HW Complexity | Power | RT Suitability |
| --- | --- | --- | --- | --- | --- |
| In-order Scalar (IMP-1 baseline) | 1 instruction issued per cycle; stall on hazards | ≤ 1.0 (≈0.72 effective) | Low | Low | Excellent — deterministic latency |
| In-order Superscalar (2-wide) | Issue 2 instructions per cycle from two aligned instruction pairs | ≤ 2.0 (≈1.4 effective) | Medium | Medium | Good — some latency variance from alignment |
| Out-of-Order (OoO) | Reorder Buffer (ROB) + reservation stations; dynamic scheduling | ≤ 4–6 (≈3.0 effective) | Very High | High | Poor — non-deterministic execution order |
| Speculative Execution | Execute past unresolved branches; rollback on mispredict | ≤ 4+ (≈3.5 effective) | Very High | Very High | Poor — speculative state complicates real-time guarantees |
| SMT (2 threads) | Two hardware threads share pipeline; fill bubbles with other thread's ops | ≤ 2.0 (≈1.8 effective) | Medium-High | Medium | Good — sensor + control threads naturally dual-threaded |

### 2.4.2 Recommendation for IMP-1

For an industrial real-time monitoring processor, deterministic latency is paramount — a late sensor alarm is worse than a slow one. Therefore:

- In-order superscalar (2-wide issue) is the recommended upgrade path. It doubles peak IPC with manageable hardware complexity and preserves instruction-order guarantees.

- OoO execution is rejected: the reorder buffer introduces variable commit latency, violating real-time deadlines.

- Speculative execution is rejected: complex rollback machinery and security vulnerabilities (Spectre-class) are unacceptable in safety-critical industrial environments.

## 2.5 SIMULTANEOUS MULTITHREADING (SMT) SUITABILITY

- SMT allows two hardware threads to share the same physical pipeline, filling pipeline slots that would otherwise be wasted during cache misses or I/O waits.

IMP-1 SMT mapping:

- Thread 0 — Sensor Acquisition Thread: Runs the ADC polling and DMA transfer loop. Frequently stalls on memory (sensor buffer writes). Creates many pipeline bubbles.

- Thread 1 — Data Processing Thread: Computes moving averages, FFT for vibration analysis, threshold comparisons. CPU-bound; rarely stalls.

- When Thread 0 stalls on a memory access (12–15% of cycles), Thread 1's instructions fill those pipeline slots — improving overall pipeline utilization from ~71% to ~88%. The estimated IPC improvement is 1.24× with 2-thread SMT.

SMT verdict for IMP-1: Suitable. The natural workload decomposition (I/O-bound acquisition + CPU-bound processing) makes IMP-1 an ideal SMT candidate. Hardware cost is moderate (duplicate PC, register file, interrupt handler; share pipeline stages and caches).

# SECTION 3: HIERARCHICAL MEMORY SYSTEM & PERFORMANCE EVALUATION

CO Mapping: CO4  |  Marks: 25

## 3.1 CACHE HIERARCHY DESIGN

- The IMP-1 employs a 3-level cache hierarchy optimized for the sensor monitoring workload's access patterns: high temporal locality (repeated threshold comparisons on the same sensor values) and moderate spatial locality (sequential sensor array traversal).

| PARAMETER | L1-I CACHE | L1-D CACHE | L2 UNIFIED | L3 UNIFIED |
| --- | --- | --- | --- | --- |
| Capacity | 32 KB | 32 KB | 256 KB | 2 MB |
| Associativity | 4-way SA | 4-way SA | 8-way SA | 16-way SA |
| Block size | 64 B | 64 B | 64 B | 64 B |
| Sets | 128 | 128 | 512 | 2048 |
| Access latency | 2 cycles | 2 cycles | 8 cycles | 30 cycles |
| Hit rate | 97% | 92% | 97% (of L1 miss) | 99% (of L2 miss) |
| Write policy | N/A (read-only) | Write-back + Allocate | Write-back + Allocate | Write-back + Allocate |
| Replacement | LRU | LRU | Pseudo-LRU | Pseudo-LRU |
| Technology | SRAM | SRAM | SRAM | SRAM |
| Power (active) | 45 mW | 45 mW | 120 mW | 400 mW |

## 3.2 AVERAGE MEMORY ACCESS TIME (AMAT) CALCULATION

### 3.2.1 Formula

AMAT = h1×t1 + (1−h1) × [h2×t2 + (1−h2) × [h3×t3 + (1−h3)×tDRAM]]

Where h = hit rate, t = access time for each level.

### 3.2.2 Step-by-Step Calculation

Given:

- t_L1 = 2 cycles, h_L1 = 0.92 (data cache)

- t_L2 = 8 cycles, h_L2 = 0.97 (hit rate of L1 misses that hit L2)

- t_L3 = 30 cycles, h_L3 = 0.99 (hit rate of L2 misses that hit L3)

- t_DRAM = 200 cycles, miss rate at L3 = 0.01

Step 1 — Innermost term (L3 miss penalty):

L3 miss penalty = t_L3 + (1 − h_L3) × t_DRAM = 30 + 0.01 × 200 = 30 + 2.0 = 32.0 cycles

Step 2 — L2 miss penalty:

L2 miss penalty = t_L2 + (1 − h_L2) × 32.0 = 8 + 0.03 × 32.0 = 8 + 0.96 = 8.96 cycles

Step 3 — AMAT:

AMAT = t_L1 + (1 − h_L1) × 8.96 = 2 + 0.08 × 8.96 = 2 + 0.717 = 2.717 cycles ≈ 5.43 ns at 500 MHz

### 3.2.3 AMAT Sensitivity Analysis

| L1 HIT RATE | AMAT (CYCLES) | AMAT (NS AT 500 MHZ) | IMPACT |
| --- | --- | --- | --- |
| 85% | 3.34 | 6.68 | Higher miss rate — degraded cache sizing or cold start |
| 90% | 2.90 | 5.80 | Moderate — possible after context switch |
| 92% (baseline) | 2.72 | 5.43 | Nominal operating point |
| 95% | 2.45 | 4.90 | Warm cache — sustained sensor-loop workload |
| 98% | 2.18 | 4.36 | Best case — tight inner loop with full reuse |

## 3.3 VIRTUAL MEMORY WITH PAGING

### 3.3.1 Page Table Configuration

The IMP-1 uses a 32-bit virtual address space with 4 KB pages:

- Virtual address: 32 bits → 20-bit VPN + 12-bit page offset

- Physical address: 32 bits → up to 4 GB physical address space (LPDDR4 bank supports 2 GB populated)

- Page Table Entry (PTE): 4 bytes per entry → page table size = 2²⁰ entries × 4 B = 4 MB (acceptable for industrial OS)

- TLB: 64-entry, fully associative, LRU replacement. TLB hit time = 1 cycle; TLB miss → page table walk = 5–10 cycles (cached page table entries in L2)

### 3.3.2 Address Translation Process

For a virtual address VA = 0x00402010 (sensor buffer access):

| STEP | OPERATION | VALUE |
| --- | --- | --- |
| 1. Extract VPN | VA[31:12] = VPN | VPN = 0x00402 |
| 2. TLB Lookup | Check TLB for VPN tag match | TLB hit (assume warm) → PFN = 0x01C00 |
| 3. Form PA | PA = PFN[31:12] ¦ VA[11:0] | PA = 0x01C00010 |
| 4. Cache Lookup | Use PA to index L1-D cache | L1-D hit (8-word aligned block) |
| 5. Return Data | Data returned to pipeline MEM stage | Latency = 1 (TLB) + 2 (L1) = 3 cycles |

### 3.3.3 TLB Effective Access Time

TLB hit rate = 98.5% (high due to repeated sensor buffer access patterns)

Effective Memory Access Time (EMAT) = TLB_hit_rate × (TLB_time + AMAT) + TLB_miss_rate × (TLB_miss_penalty + AMAT)

EMAT = 0.985 × (1 + 2.72) + 0.015 × (8 + 2.72) = 0.985 × 3.72 + 0.015 × 10.72 = 3.66 + 0.16 = 3.82 cycles

## 3.4 MEMORY TECHNOLOGY COMPARISON

A comprehensive comparison of all five memory technologies relevant to the IMP-1 memory hierarchy:

| PROPERTY | SRAM | DRAM (LPDDR4) | NAND FLASH | MRAM | RRAM |
| --- | --- | --- | --- | --- | --- |
| Technology basis | Flip-flop (6T cell) | Capacitor + transistor | Floating-gate transistor | Magnetic tunnel junction | Resistive switching oxide |
| Read latency | 1–5 ns | 40–60 ns | 25–100 µs | 2–10 ns | 10–50 ns |
| Write latency | 1–5 ns | 40–60 ns | 200–1500 µs | 2–10 ns | 10–50 ns |
| Endurance (cycles) | >10¹⁵ | >10¹⁵ | 10³ – 10⁵ | >10¹² | 10⁶ – 10¹² |
| Bandwidth | >100 GB/s | 51.2 GB/s | 0.5–3 GB/s | >10 GB/s | 1–10 GB/s |
| Non-volatile? | No | No | Yes | Yes | Yes |
| Cell size (F²) | 60–120 | 6–8 | 4–8 | 20–40 | 4–10 |
| Density | Low | High | Very High | Medium | High (emerging) |
| Active power | Medium | Medium | Low (sleep) | Very Low | Very Low |
| Standby power | Low | Medium (refresh) | Zero | Zero | Zero |
| Cost (USD/GB) | ~$5,000 | ~$5 | ~$0.10 | ~$50 | ~$10 (projected) |
| Maturity | Mature | Mature | Mature | Emerging production | Early production |
| IMP-1 role | L1, L2, L3 cache | Main memory | Sensor event log | L3 / NV-cache candidate | Fast event buffer (future) |

### 3.4.1 TECHNOLOGY SELECTION RATIONALE

- SRAM for L1/L2/L3: The only technology meeting the 2–30 cycle latency requirements of on-chip cache. High cost is acceptable at KB–MB scales.

- LPDDR4 DRAM for Main Memory: Industry standard; lowest cost/GB for volatile bulk storage; 51.2 GB/s bandwidth handles DMA burst transfers from 8 sensor channels simultaneously.

- NAND Flash for Sensor Event Log: Non-volatile long-term storage for sensor history, alarm logs, and firmware. Low write endurance (10⁵ cycles) is manageable with wear-leveling and infrequent writes.

- MRAM as NV-Cache (recommended future upgrade): Near-SRAM speeds with non-volatility; ideal for critical sensor state that must survive power loss. Current cost is the barrier.

- RRAM: Promising for high-density non-volatile storage with DRAM-like speeds, but currently too immature for deployment in safety-critical industrial systems.

# SECTION 4: RESULTS, COMPARISON & ENGINEERING RECOMMENDATION

CO MAPPING: CO1, CO3, CO4  |  MARKS: 10

## 4.1 CONSOLIDATED RESULTS SUMMARY

| METRIC | UNPIPELINED BASELINE | PIPELINED (IN-ORDER) | PIPELINED + SMT 2T | UNIT |
| --- | --- | --- | --- | --- |
| Clock Frequency | 500 | 500 | 500 | MHz |
| CPI | 4.00 | 1.39 | 1.12 | cycles/instruction |
| IPC | 0.25 | 0.72 | 0.89 | instructions/cycle |
| Execution Time (10K reads) | 800 | 277.6 | 224.0 | µs |
| Throughput | 125 | 360 | 446 | MIPS |
| AMAT | 200.0 | 2.72 | 2.72 | cycles |
| EMAT (with TLB) | 201.0 | 3.82 | 3.82 | cycles |
| Pipeline Utilization | — | 71% | 88% | % |
| Branch Prediction Accuracy | — | 88% | 88% | % |
| Estimated Core Power | 8.0 | 5.0 | 5.5 | W |
| Performance/Watt | 15.6 | 72.1 | 81.1 | MIPS/W |

## 4.2 BOTTLENECK IDENTIFICATION

Analysis of the baseline and optimized designs identifies the following key bottlenecks and their impact:

| BOTTLENECK | IMPACT | MITIGATION APPLIED | RESIDUAL IMPACT |
| --- | --- | --- | --- |
| Load-use RAW hazards | +0.025 CPI (stall cycles) | MEM/WB forwarding + 1-cycle stall | Minimal — 1 stall per load-use occurrence |
| Branch mispredictions | +0.048 CPI | 2-bit BHT predictor (88% accuracy) | Moderate — 12% mispredict rate on irregular data |
| L1-D cache misses (8%) | AMAT increases 0.717 cycles above L1 hit time | 4-way SA, LRU, 64B blocks; prefetcher | Small — L2 absorbs 97% of L1 misses in 8 cycles |
| DRAM latency (200 cycles) | Catastrophic for frequent off-chip accesses | L3 cache reduces DRAM accesses to <0.024% of all refs | Negligible with warm cache |
| Pipeline bubbles from I/O stalls | SMT Thread 0 stalls 12–15% of cycles | SMT: Thread 1 fills bubbles | Resolved by SMT — utilization 71%→88% |
| Sensor bus bandwidth | SPI @ 10 MHz = 1.25 MB/s per channel | DMA controller offloads CPU; 8 channels parallel | Not a bottleneck at 8 channels × 12-bit @ 1 MSPS |

## 4.3 OPTIMIZED SYSTEM ARCHITECTURE RECOMMENDATION

### 4.3.1 Processor Recommendation

Recommended Design: In-order 5-stage pipeline with 2-thread SMT, operating at 500 MHz

Justification:

- Performance: 2.88× faster than unpipelined baseline; SMT adds a further 1.24× utilization gain at modest hardware cost.

- Determinism: In-order execution guarantees bounded worst-case latency — critical for ISO 13849 functional safety compliance in industrial environments.

- Power Efficiency: 5.5W with SMT vs 8W baseline = 31% power reduction; 81.1 MIPS/W vs 15.6 MIPS/W = 5.2× energy efficiency improvement.

- Cost: In-order pipeline with SMT avoids the silicon area and verification cost of a full OoO engine.

### 4.3.2 Memory Hierarchy Recommendation

| LAYER | TECHNOLOGY | SIZE | JUSTIFICATION |
| --- | --- | --- | --- |
| L1-I / L1-D Cache | SRAM (6T) | 32 KB each | 2-cycle latency; split Harvard eliminates structural hazards |
| L2 Unified Cache | SRAM (6T) | 256 KB | 8-cycle latency; absorbs 97% of L1 misses |
| L3 Shared Cache | SRAM (6T) | 2 MB | 30-cycle latency; AMAT = 2.72 cycles with L3 |
| Main Memory | LPDDR4 DRAM | 2 GB | Lowest cost/GB; 51.2 GB/s for DMA sensor bursts |
| Non-volatile Log | NAND Flash | 64 GB | Long-term sensor history; non-volatile; cost ~$6.40 |
| Critical State NV Buffer (future) | MRAM | 8 MB | Near-zero power; survives power loss; alarm state persistence |

### 4.3.3 Scalability & Reliability Considerations

- Scalability: The 2-thread SMT design can be extended to 4-thread SMT for future sensor expansion (16+ channels) with moderate register file and TLB duplication.

- Reliability: SRAM caches support ECC (SECDED — Single Error Correct, Double Error Detect) for harsh EMI-exposed industrial environments. LPDDR4 also supports ECC mode.

- Power Management: Dynamic Voltage and Frequency Scaling (DVFS) reduces core voltage from 1.0V to 0.75V during low-activity periods (sensor standby), cutting dynamic power by ~44%.

- Thermal: Total TDP ≤ 5W fits within passive cooling budget for a DIN-rail mounted industrial controller, eliminating fan reliability concerns.

# SECTION 5: DESIGN DECISIONS, SDG RELEVANCE & LEARNING OUTCOMES

CO MAPPING: CO1, CO3, CO4  |  MARKS: 10

## 5.1 DESIGN DECISION JUSTIFICATION

### 5.1.1 Why In-Order Pipeline Over OoO?

- Out-of-order execution maximizes average-case IPC but introduces non-deterministic commit latency. For an industrial safety system where a missed sensor alarm could cause equipment damage or injury, worst-case latency bounds are more important than average throughput. The in-order 5-stage pipeline guarantees that instruction N completes before instruction N+1 commits, making formal timing verification tractable under IEC 61508 (functional safety standard).

### 5.1.2 Why 2-Bit BHT Over Simpler 1-Bit?

- The sensor monitoring loop has a dominant back-edge branch (loop repeat) that the 1-bit predictor would mis-predict on every loop entry and exit (2 mispredictions per loop iteration). The 2-bit saturating counter reaches "Strongly Taken" after 2 iterations and stays there, achieving 96%+ accuracy for the back-edge branch. The 88% overall accuracy reflects occasional forward branches (alarm condition triggers) that are inherently unpredictable.

### 5.1.3 Why 4-Way Set-Associative L1 vs Direct-Mapped?

- Direct-mapped caches suffer from conflict misses when two frequently accessed sensor buffers map to the same cache set. A 32 KB direct-mapped cache with 64B blocks has 512 sets — two sensor arrays 32 KB apart would evict each other on every access. 4-way associativity provides 4 "ways" to hold conflicting addresses, reducing conflict misses by approximately 60% for the sensor-array access pattern, at modest area cost (4× comparison logic per set).

### 5.1.4 Why LPDDR4 Over Standard DDR4 for Main Memory?

- LPDDR4 (Low Power DDR4) operates at 0.6V core voltage vs DDR4's 1.2V, reducing memory subsystem power by approximately 50% while delivering comparable bandwidth (51.2 GB/s in dual-channel). For a 24/7 industrial monitoring node, this translates to significant energy savings over the product lifetime and extends the viable operating temperature range.

## 5.2 Challenges Encountered

- AMAT Calculation Complexity: Initially confused between cumulative and conditional miss rates. Resolved by explicitly writing the recursive AMAT formula and computing innermost-first.

- Hazard Classification: Distinguishing WAR (Write-After-Read) from RAW in the out-of-order context — for in-order pipelines, WAR hazards cannot occur since instructions issue in program order.

- SMT vs Superscalar Trade-off: The decision between 2-wide superscalar and 2-thread SMT required careful analysis. SMT was chosen because the IMP-1 workload is I/O-mixed (not instruction-level parallel in a single thread), making SMT more efficient at filling pipeline slots.

- Memory Technology Selection: Balancing MRAM's performance advantages against its 10× cost premium over NAND Flash required defining clear use-case boundaries — MRAM only for critical non-volatile state (alarm history, configuration), not bulk sensor logs.

## 5.3 SDG ALIGNMENT

### 5.3.1 SDG 7 — Affordable and Clean Energy

The IMP-1 design directly contributes to SDG 7 through multiple energy reduction mechanisms:

- 31% total power reduction (8W → 5.5W) through pipeline efficiency improvements reduces energy consumption by 2,628 Wh per year per monitoring node (assuming continuous operation).

- DVFS support enables further 44% dynamic power reduction during sensor standby periods, enabling deployment on energy harvesting (solar/vibration) power supplies — eliminating grid dependence at remote monitoring sites.

- LPDDR4's 50% lower memory power vs DDR4 and MRAM's near-zero standby power support ultra-low-power operating modes (< 1W), enabling battery-backed operation for up to 72 hours during grid outages.

- 5.2× improvement in performance-per-watt (15.6 → 81.1 MIPS/W) means more industrial sensing and control can be accomplished with less electrical energy — directly aligned with SDG 7.3 (doubling global energy efficiency by 2030).

### 5.3.2 SDG 9 — Industry, Innovation and Infrastructure

The IMP-1 architecture supports SDG 9 through enabling smarter industrial infrastructure:

- Real-time sensor processing at 360+ MIPS enables predictive maintenance analytics at the edge — detecting bearing failures, thermal anomalies, and pressure leaks before they cause costly downtime.

- The SMT design supports concurrent I/O handling and computation, enabling IMP-1 to replace two separate microcontrollers (one for I/O, one for computation) in existing installations — reducing hardware BOM and manufacturing waste.

- The hierarchical memory system enables local machine learning inference (lightweight neural networks for anomaly detection) without cloud round-trips, improving infrastructure resilience in factory environments with unreliable connectivity.

- Open, standards-compliant RISC ISA design enables academic and SME access to the architecture for research and derivative products — supporting inclusive industrial innovation.

### 5.3.3 SDG 12 — Responsible Consumption and Production

The design addresses SDG 12 through responsible resource utilization:

- Right-sized memory hierarchy: Rather than provisioning maximum possible cache sizes, the L1/L2/L3 design is analytically sized (AMAT calculation) to the actual workload requirements — avoiding silicon area waste and reducing manufacturing energy per device.

- NAND Flash for sensor logs reduces server infrastructure needs: 64 GB on-device log storage eliminates the need for continuous cloud streaming, reducing data center energy consumption and network bandwidth consumption.

- MRAM's non-volatility eliminates the need for battery-backed SRAM (super-capacitor + SRAM) in existing designs for critical state retention — removing hazardous materials (lithium batteries) from the BOM.

## 5.4 Learning Outcomes

This project developed the following competencies:

- Quantitative performance analysis: Derived CPI, AMAT, and EMAT from first principles, reinforcing the connection between architectural decisions and measurable performance metrics.

- Hazard taxonomy: The process of manually tracing instructions through the pipeline stages clarified the distinction between RAW, WAW, WAR, structural, and control hazards — concepts that were previously understood abstractly.

- Technology trade-off reasoning: Comparing SRAM/DRAM/NAND/MRAM/RRAM across five dimensions (latency, bandwidth, endurance, volatility, cost) developed the multi-criteria engineering decision framework applicable to future system design tasks.

- SDG integration: Connecting architectural choices (DVFS, LPDDR4, MRAM) to specific SDG sub-targets demonstrated that energy-aware design is not a constraint but a design goal with measurable societal benefit.

## 5.5 Possible Improvements & Future Work

- Hardware Prefetcher: Adding a stride-based prefetcher for the sensor array traversal pattern could improve L1-D hit rate from 92% to ~96%, reducing AMAT from 2.72 to ~2.32 cycles.

- RISC-V ISA Migration: Adopting the open RISC-V ISA would enable access to an extensive open-source toolchain (GCC, LLVM, OpenOCD) and IP ecosystem, reducing development cost.

- 4-Thread SMT Upgrade: Expanding to 4 hardware threads could support concurrent execution of acquisition, processing, communication, and watchdog threads on a single core.

- Near-Memory Computing: Placing simple processing elements adjacent to DRAM banks could perform sensor data aggregation before DMA transfer to the CPU, reducing memory bandwidth consumption by up to 60%.

- MRAM L3 Cache Prototype: Evaluating an MRAM-backed L3 cache in a test system would quantify the real-world power and latency benefits vs SRAM-based L3 for the IMP-1 workload.

## 5.6 CONCLUSION

The IMP-1 industrial monitoring processor demonstrates that energy efficiency and real-time performance are complementary, not competing, design goals. By combining a 5-stage in-order pipeline with 2-thread SMT, a 3-level SRAM cache hierarchy analytically sized via AMAT calculations, LPDDR4 main memory, and a hybrid non-volatile storage strategy (NAND Flash + MRAM roadmap), the design achieves:

- 2.88× performance improvement over the unpipelined baseline

- 5.2× better energy efficiency (MIPS/W)

- Real-time determinism (bounded worst-case latency) suitable for ISO 13849 / IEC 61508 functional safety certification

- Alignment with SDG 7, 9, and 12 through energy reduction, industrial innovation enablement, and responsible resource consumption

The design provides a solid foundation for a production industrial monitoring SoC and a clear upgrade roadmap (MRAM integration, 4T-SMT, hardware prefetcher) as the technology matures.

# APPENDIX A: QUICK-REFERENCE PERFORMANCE CALCULATIONS

## A.1 CPI Derivation — Full Working

| INSTRUCTION MIX | FRACTION F | CPI_I | F × CPI_I |
| --- | --- | --- | --- |
| ALU / R-type | 0.35 | 1.00 | 0.3500 |
| Load/Store (w/ 8% miss, 2-cycle penalty) | 0.25 | 1.16 | 0.2900 |
| Branch (w/ 12% mispredict, 2-cycle penalty) | 0.20 | 1.24 | 0.2480 |
| I/O Read (sensor poll) | 0.15 | 2.00 | 0.3000 |
| Multiply / Divide | 0.05 | 4.00 | 0.2000 |
| TOTAL | 1.00 | — | 1.3880 |

## A.2 AMAT Derivation — Annotated

| STEP | EXPRESSION | VALUE |
| --- | --- | --- |
| L3 miss penalty | t_L3 + (1 − h_L3) × t_DRAM | 30 + (0.01 × 200) = 32.00 cycles |
| L2 miss penalty | t_L2 + (1 − h_L2) × 32.00 | 8 + (0.03 × 32) = 8.96 cycles |
| AMAT (data) | t_L1 + (1 − h_L1) × 8.96 | 2 + (0.08 × 8.96) = 2.717 cycles |
| AMAT in ns (500 MHz) | 2.717 × 2 ns/cycle | 5.43 ns |
| EMAT (with TLB, 98.5% hit) | 0.985×(1+2.72) + 0.015×(8+2.72) | 3.66 + 0.16 = 3.82 cycles |

## A.3 Execution Time Summary

| PARAMETER | VALUE | FORMULA / SOURCE |
| --- | --- | --- |
| Instruction Count (IC) | 100,000 instructions | Workload definition: 10,000 sensor reads × 10 instructions |
| Weighted CPI | 1.388 | Σ(f_i × CPI_i) — see A.1 |
| Total Clock Cycles | 138,800 cycles | IC × CPI |
| Clock Period | 2 ns | 1 / 500 MHz |
| Execution Time | 277.6 µs | Cycles × Period |
| Throughput | 360.3 MIPS | IC / Execution Time / 10⁶ |
| Performance/Watt | 72.1 MIPS/W | MIPS / 5W |
| Energy per 10K reads | 1.388 µJ | Power × Time = 5W × 277.6µs |

## A.4 Abbreviations

| TERM | EXPANSION |
| --- | --- |
| ALU | Arithmetic Logic Unit |
| AMAT | Average Memory Access Time |
| BHT | Branch History Table |
| CPI | Cycles Per Instruction |
| DRAM | Dynamic Random Access Memory |
| DVFS | Dynamic Voltage and Frequency Scaling |
| EMAT | Effective Memory Access Time (including TLB) |
| IPC | Instructions Per Cycle |
| IMP-1 | Industrial Monitoring Processor — revision 1 |
| LPDDR4 | Low Power Double Data Rate 4 DRAM |
| MRAM | Magnetoresistive RAM |
| NAND Flash | NAND-gate Flash non-volatile memory |
| OoO | Out-of-Order execution |
| PC | Program Counter |
| RAW | Read After Write (data hazard) |
| RRAM | Resistive RAM |
| SA | Set Associative |
| SMT | Simultaneous Multithreading |
| SRAM | Static Random Access Memory |
| TLB | Translation Lookaside Buffer |
| VPN | Virtual Page Number |
| WB | Write Back (pipeline stage or cache policy) |

OUTPUT

PERFORMANCE ANALYSIS:-