"""

Section 2: Pipelined Processor Organization & Hazard Analysis

CSA1203 – Computer Architecture | IMP-1 Industrial Monitoring Processor

CO Mapping: CO3 | Marks: 30

Implements:

- 5-stage in-order pipeline (IF / ID / EX / MEM / WB)

- Pipeline registers (IF/ID, ID/EX, EX/MEM, MEM/WB)

- RAW hazard detection & forwarding unit

- Load-use stall insertion

- 2-bit saturating counter branch predictor (BHT)

- Structural hazard analysis

- Timing diagram generator

- Superscalar / OoO / SMT comparison

"""

from dataclasses import dataclass, field

from typing import Optional, List, Dict, Tuple

from enum import Enum, auto

# ─────────────────────────────────────────────────────────────

# 2.0  Instruction representation

# ─────────────────────────────────────────────────────────────

class InstType(Enum):

ALU    = "ALU"

LOAD   = "LOAD"

STORE  = "STORE"

BRANCH = "BRANCH"

JUMP   = "JUMP"

IO     = "IO_READ"

MUL    = "MULTIPLY"

NOP    = "NOP"

@dataclass

class Instruction:

label: str                  # human-readable name, e.g. "LW R1,0(R2)"

itype: InstType

dest:  Optional[str] = None # destination register

src1:  Optional[str] = None

src2:  Optional[str] = None

imm:   int = 0              # immediate / offset

def __str__(self):

return self.label

# ─────────────────────────────────────────────────────────────

# 2.1  Pipeline Registers

# ─────────────────────────────────────────────────────────────

@dataclass

class IFIDReg:

inst:  Optional[Instruction] = None

pc:    int = 0

@dataclass

class IDEXReg:

inst:      Optional[Instruction] = None

rs1_val:   int = 0

rs2_val:   int = 0

rd:        Optional[str] = None

imm:       int = 0

alu_src:   bool = False

mem_read:  bool = False

mem_write: bool = False

reg_write: bool = False

is_branch: bool = False

@dataclass

class EXMEMReg:

inst:      Optional[Instruction] = None

alu_result: int = 0

rd:        Optional[str] = None

rs2_val:   int = 0

mem_read:  bool = False

mem_write: bool = False

reg_write: bool = False

branch_taken: bool = False

branch_target: int = 0

@dataclass

class MEMWBReg:

inst:       Optional[Instruction] = None

rd:         Optional[str] = None

result:     int = 0         # ALU result or loaded memory data

reg_write:  bool = False

# ─────────────────────────────────────────────────────────────

# 2.2  2-bit Saturating Counter Branch Predictor (BHT)

# ─────────────────────────────────────────────────────────────

class BHTPredictor:

"""

512-entry Branch History Table with 2-bit saturating counters.

States: 0=Strongly NT, 1=Weakly NT, 2=Weakly T, 3=Strongly T

"""

STRONGLY_NT = 0

WEAKLY_NT   = 1

WEAKLY_T    = 2

STRONGLY_T  = 3

def __init__(self, entries: int = 512):

self.table = [self.WEAKLY_NT] * entries   # start neutral

self.hits  = 0

self.total = 0

def _index(self, pc: int) -> int:

return (pc >> 2) % len(self.table)

def predict(self, pc: int) -> bool:

"""Returns True (predict Taken) if counter ≥ 2."""

return self.table[self._index(pc)] >= self.WEAKLY_T

def update(self, pc: int, actually_taken: bool):

idx = self._index(pc)

old = self.table[idx]

if actually_taken:

self.table[idx] = min(self.STRONGLY_T, old + 1)

else:

self.table[idx] = max(self.STRONGLY_NT, old - 1)

predicted = (old >= self.WEAKLY_T)

self.total += 1

if predicted == actually_taken:

self.hits += 1

@property

def accuracy(self) -> float:

return self.hits / self.total if self.total else 0.0

def state_name(self, pc: int) -> str:

names = {0:"Strongly NT", 1:"Weakly NT", 2:"Weakly T", 3:"Strongly T"}

return names[self.table[self._index(pc)]]

# ─────────────────────────────────────────────────────────────

# 2.3  Register File

# ─────────────────────────────────────────────────────────────

class RegisterFile:

def __init__(self):

self.regs: Dict[str, int] = {f"R{i}": 0 for i in range(32)}

self.regs["R0"] = 0  # hardwired

def read(self, reg: str) -> int:

if reg is None:

return 0

return self.regs.get(reg, 0)

def write(self, reg: str, val: int):

if reg and reg != "R0":

self.regs[reg] = val & 0xFFFFFFFF   # 32-bit mask

# ─────────────────────────────────────────────────────────────

# 2.4  Simple Data Memory

# ─────────────────────────────────────────────────────────────

class DataMemory:

def __init__(self):

self.mem: Dict[int, int] = {}

# Pre-load sensor data at 0x2000+

for i in range(32):

self.mem[0x2000 + i*4] = 0x100 + i    # simulated sensor values

def load(self, addr: int) -> int:

return self.mem.get(addr & ~0x3, 0)        # word-aligned

def store(self, addr: int, val: int):

self.mem[addr & ~0x3] = val & 0xFFFFFFFF

# ─────────────────────────────────────────────────────────────

# 2.5  5-Stage Pipeline Simulator

# ─────────────────────────────────────────────────────────────

class Pipeline5Stage:

"""

Simulates the IMP-1 5-stage in-order pipeline.

Tracks: stalls, forwards, mispredictions, timing diagrams.

"""

def __init__(self):

self.rf  = RegisterFile()

self.mem = DataMemory()

self.bht = BHTPredictor(512)

# Pipeline registers

self.if_id  = IFIDReg()

self.id_ex  = IDEXReg()

self.ex_mem = EXMEMReg()

self.mem_wb = MEMWBReg()

# Counters

self.cycle        = 0

self.stalls       = 0

self.forwards_ex  = 0

self.forwards_mem = 0

self.flushes      = 0

# Timeline recording for diagram: {inst_label: {stage: cycle}}

self.timeline: Dict[str, Dict[str, int]] = {}

# ── helpers ────────────────────────────────────────────────

def _record(self, stage: str, inst: Optional[Instruction]):

if inst and inst.itype != InstType.NOP:

if inst.label not in self.timeline:

self.timeline[inst.label] = {}

self.timeline[inst.label][stage] = self.cycle

def _is_load_use_hazard(self) -> bool:

"""

Detect load-use hazard:

ID/EX register has a LOAD that writes to a register

that the next instruction (currently in IF/ID) reads.

"""

if not self.id_ex.mem_read:

return False

incoming = self.if_id.inst

if incoming is None or incoming.itype == InstType.NOP:

return False

rd = self.id_ex.rd

return rd in (incoming.src1, incoming.src2)

def _forwarding(self, rs: Optional[str]) -> Tuple[int, str]:

"""

Return forwarded value and source label for register rs.

Priority: EX/MEM > MEM/WB > Register File.

"""

if rs is None:

return 0, "RF"

if self.ex_mem.reg_write and self.ex_mem.rd == rs:

self.forwards_ex += 1

return self.ex_mem.alu_result, "EX/MEM"

if self.mem_wb.reg_write and self.mem_wb.rd == rs:

self.forwards_mem += 1

return self.mem_wb.result, "MEM/WB"

return self.rf.read(rs), "RF"

def _alu(self, itype: InstType, a: int, b: int) -> int:

if itype in (InstType.ALU, InstType.LOAD, InstType.STORE):

return (a + b) & 0xFFFFFFFF

if itype == InstType.MUL:

return (a * b) & 0xFFFFFFFF

if itype == InstType.BRANCH:

return a - b                            # compare for equality

return 0

# ── execute one clock cycle ─────────────────────────────────

def tick(self, next_inst: Optional[Instruction], pc: int):

self.cycle += 1

stall_this_cycle = False

# ── WB ──────────────────────────────────────────────────

if self.mem_wb.reg_write and self.mem_wb.rd:

self.rf.write(self.mem_wb.rd, self.mem_wb.result)

self._record("WB", self.mem_wb.inst)

# ── MEM ─────────────────────────────────────────────────

new_mem_wb = MEMWBReg(

inst      = self.ex_mem.inst,

rd        = self.ex_mem.rd,

reg_write = self.ex_mem.reg_write

)

if self.ex_mem.mem_read:

new_mem_wb.result = self.mem.load(self.ex_mem.alu_result)

elif self.ex_mem.mem_write:

self.mem.store(self.ex_mem.alu_result, self.ex_mem.rs2_val)

new_mem_wb.result = 0

else:

new_mem_wb.result = self.ex_mem.alu_result

self._record("MEM", self.ex_mem.inst)

# ── EX ──────────────────────────────────────────────────

a, src_a = self._forwarding(self.id_ex.inst.src1 if self.id_ex.inst else None)

if self.id_ex.alu_src:

b, src_b = self.id_ex.imm, "IMM"

else:

b, src_b = self._forwarding(self.id_ex.inst.src2 if self.id_ex.inst else None)

alu_out = self._alu(

self.id_ex.inst.itype if self.id_ex.inst else InstType.NOP,

a, b

)

# Branch resolution

branch_taken   = False

branch_target  = 0

if self.id_ex.is_branch:

branch_taken  = (alu_out == 0)           # BEQ: rs1==rs2

branch_target = self.id_ex.imm            # pre-computed target

predicted     = self.bht.predict(self.if_id.pc)

if predicted != branch_taken:

self.flushes += 1                     # flush 2 instructions

self.bht.update(self.if_id.pc, branch_taken)

new_ex_mem = EXMEMReg(

inst          = self.id_ex.inst,

alu_result    = alu_out,

rd            = self.id_ex.rd,

rs2_val       = b,

mem_read      = self.id_ex.mem_read,

mem_write     = self.id_ex.mem_write,

reg_write     = self.id_ex.reg_write,

branch_taken  = branch_taken,

branch_target = branch_target

)

self._record("EX", self.id_ex.inst)

# ── ID ──────────────────────────────────────────────────

# Load-use hazard check

stall_this_cycle = self._is_load_use_hazard()

if stall_this_cycle:

self.stalls += 1

new_id_ex = IDEXReg()           # bubble

else:

inst = self.if_id.inst

if inst is None:

new_id_ex = IDEXReg()

else:

new_id_ex = IDEXReg(

inst      = inst,

rs1_val   = self.rf.read(inst.src1),

rs2_val   = self.rf.read(inst.src2),

rd        = inst.dest,

imm       = inst.imm,

alu_src   = inst.itype in (InstType.LOAD, InstType.STORE, InstType.IO),

mem_read  = inst.itype == InstType.LOAD,

mem_write = inst.itype == InstType.STORE,

reg_write = inst.itype in (InstType.ALU, InstType.LOAD,

InstType.IO,  InstType.MUL),

is_branch = inst.itype == InstType.BRANCH

)

self._record("ID", self.if_id.inst if not stall_this_cycle else None)

# ── IF ──────────────────────────────────────────────────

if stall_this_cycle:

new_if_id = self.if_id         # freeze IF/ID

else:

new_if_id = IFIDReg(inst=next_inst, pc=pc)

self._record("IF", next_inst if not stall_this_cycle else None)

# Commit pipeline registers

self.if_id  = new_if_id

self.id_ex  = new_id_ex

self.ex_mem = new_ex_mem

self.mem_wb = new_mem_wb

return stall_this_cycle

# ── run a full instruction sequence ────────────────────────

def run(self, instructions: List[Instruction]):

"""

Drive the pipeline with a list of instructions.

Pads with NOPs to drain the pipeline at the end.

"""

NOP = Instruction("NOP", InstType.NOP)

program = instructions + [NOP] * 5    # drain pipeline

ip = 0                                # instruction pointer

pc = 0x1000

while ip < len(program):

fetch_inst = program[ip] if ip < len(program) else NOP

stalled = self.tick(fetch_inst, pc)

if not stalled:

ip += 1

pc += 4

# ── timing diagram ─────────────────────────────────────────

def print_timing_diagram(self, instructions: List[Instruction]):

stage_map = {"IF": "IF", "ID": "ID", "EX": "EX", "MEM": "MEM", "WB": "WB"}

max_cycle = max(

(max(v.values()) for v in self.timeline.values() if v),

default=0

)

print("\n" + "="*80)

print("  PIPELINE TIMING DIAGRAM")

print("="*80)

header = f"  {'Instruction':<30}" + "".join(f" C{c:<3}" for c in range(1, max_cycle+2))

print(header)

print("  " + "-"*75)

for inst in instructions:

lbl = inst.label

stages = self.timeline.get(lbl, {})

row = f"  {lbl:<30}"

for c in range(1, max_cycle+2):

cell = "    "

for stage, cyc in stages.items():

if cyc == c:

abbr = stage[:2] if stage == "MEM" else stage

cell = f" {stage:<3}"

row += cell

print(row)

print()

# ─────────────────────────────────────────────────────────────

# 2.6  Hazard Analysis Report

# ─────────────────────────────────────────────────────────────

def hazard_analysis_report(pipeline: Pipeline5Stage):

print("\n" + "="*70)

print("  HAZARD ANALYSIS REPORT")

print("="*70)

total_instructions = sum(

1 for lbl, stages in pipeline.timeline.items()

if stages

)

print(f"\n  Total instructions executed : {total_instructions}")

print(f"  Load-use stalls inserted    : {pipeline.stalls}")

print(f"  Branch mispredictions       : {pipeline.flushes}")

print(f"  EX/MEM forwards applied     : {pipeline.forwards_ex}")

print(f"  MEM/WB forwards applied     : {pipeline.forwards_mem}")

print(f"  Total cycles elapsed        : {pipeline.cycle}")

cpi = pipeline.cycle / total_instructions if total_instructions else 0

print(f"  Effective CPI               : {cpi:.3f}")

print(f"  BHT accuracy                : {pipeline.bht.accuracy*100:.1f}%")

print("\n  Data Hazard Summary (RAW):")

print("  ┌──────────────────────────────────────────────┬────────────────────┐")

print("  │ Hazard Type                                  │ Resolution         │")

print("  ├──────────────────────────────────────────────┼────────────────────┤")

print("  │ ALU→ALU (EX produces, next EX reads)         │ EX/MEM forwarding  │")

print("  │ Load-Use (LOAD EX produces, next EX reads)   │ 1-cycle stall+fwd  │")

print("  │ ALU→ALU 2nd behind (MEM produces, EX reads) │ MEM/WB forwarding  │")

print("  │ WAW / WAR (write-after-write/read)           │ In-order prevents  │")

print("  └──────────────────────────────────────────────┴────────────────────┘")

print("\n  Control Hazard Summary (Branch):")

print("  2-bit saturating counter BHT (512 entries)")

print("  Misprediction penalty = 2 cycles (flush 2 fetched instructions)")

print(f"  Accuracy = {pipeline.bht.accuracy*100:.1f}% | Flushes = {pipeline.flushes}")

print("\n  Structural Hazard Summary:")

print("  IF vs MEM memory conflict → Resolved by Harvard split L1-I / L1-D")

print("  Register file WB+ID same cycle → Dual-port RF (write-first convention)")

print("  MUL multi-cycle blocks EX → Separate 4-stage multiply pipeline")

# ─────────────────────────────────────────────────────────────

# 2.7  Execution Technique Comparison

# ─────────────────────────────────────────────────────────────

def execution_technique_comparison():

techniques = [

{

"name":       "In-order Scalar (IMP-1 baseline)",

"mechanism":  "1 inst/cycle; stall on hazard",

"ipc":        "≤1.0  (~0.72 effective)",

"hw_cost":    "Low",

"power":      "Low",

"rt_suit":    "Excellent – deterministic latency",

"recommended": True

},

{

"name":       "In-order Superscalar 2-wide",

"mechanism":  "2 insts/cycle from aligned pairs",

"ipc":        "≤2.0  (~1.4 effective)",

"hw_cost":    "Medium",

"power":      "Medium",

"rt_suit":    "Good – minor alignment variance",

"recommended": True

},

{

"name":       "Out-of-Order (OoO)",

"mechanism":  "ROB + reservation stations; dynamic schedule",

"ipc":        "≤4–6 (~3.0 effective)",

"hw_cost":    "Very High",

"power":      "High",

"rt_suit":    "POOR – non-deterministic commit order",

"recommended": False

},

{

"name":       "Speculative Execution",

"mechanism":  "Execute past unresolved branches; rollback",

"ipc":        "≤4+  (~3.5 effective)",

"hw_cost":    "Very High",

"power":      "Very High",

"rt_suit":    "POOR – Spectre-class security risk",

"recommended": False

},

{

"name":       "SMT 2-thread",

"mechanism":  "Two HW threads share pipeline",

"ipc":        "≤2.0 (~1.8 effective)",

"hw_cost":    "Medium-High",

"power":      "Medium",

"rt_suit":    "Good – I/O+compute thread separation",

"recommended": True

},

]

print("\n" + "="*90)

print("  EXECUTION TECHNIQUE COMPARISON (Section 2.4)")

print("="*90)

fmt = "  {:<30} {:^14} {:^12} {:^10} {:^14} {:^6}"

print(fmt.format("Technique", "IPC potential", "HW cost", "Power", "RT Suitable", "Pick?"))

print("  " + "-"*85)

for t in techniques:

pick = "✓" if t["recommended"] else "✗"

print(fmt.format(

t["name"][:30], t["ipc"], t["hw_cost"], t["power"], t["rt_suit"][:14], pick

))

print("\n  Recommendation: In-order pipeline + 2-thread SMT (sensor I/O + compute)")

print("  OoO and Speculative execution REJECTED – violate real-time determinism.")

# ─────────────────────────────────────────────────────────────

# 2.8  SMT Analysis

# ─────────────────────────────────────────────────────────────

def smt_analysis():

thread0_stall_rate = 0.13          # sensor I/O thread stalls 13% of cycles

thread1_compute    = 1 - 0.02     # compute thread stalls only 2%

baseline_util      = 1 - thread0_stall_rate

smt_util           = baseline_util + thread0_stall_rate * thread1_compute

print("\n" + "="*70)

print("  SMT ANALYSIS (Section 2.5)")

print("="*70)

print(f"\n  Thread 0 (Sensor I/O)  : stalls {thread0_stall_rate*100:.0f}% of cycles (DMA/memory waits)")

print(f"  Thread 1 (Processing)  : stalls  ~2% of cycles (CPU-bound)")

print(f"\n  Baseline utilisation   : {baseline_util*100:.0f}%")

print(f"  SMT utilisation        : {smt_util*100:.0f}%  (+{(smt_util-baseline_util)*100:.0f}pp)")

ipc_gain = smt_util / baseline_util

print(f"  Estimated IPC gain     : {ipc_gain:.2f}×")

print(f"\n  Verdict: SMT SUITABLE — natural workload decomposition (I/O + compute)")

# ─────────────────────────────────────────────────────────────

# Main

# ─────────────────────────────────────────────────────────────

def build_sensor_program() -> List[Instruction]:

"""

Sensor monitoring inner loop (6 key instructions from the report):

I1: LW R1, 0(R2)        ← load sensor reading

I2: ADD R3, R1, R4      ← accumulate (RAW hazard on R1 → load-use stall)

I3: SW R3, 4(R2)        ← store result

I4: BEQ R3, R5, loop    ← compare against threshold → branch

I5: ADDI R6, R0, 1      ← set alarm flag

I6: LW R7, 8(R2)        ← load next sensor register

"""

R = lambda name, default=0: name  # register name helper

instructions = [

Instruction("LW  R1,0(R2)",   InstType.LOAD,   dest="R1", src1="R2", imm=0x2000),

Instruction("ADD R3,R1,R4",   InstType.ALU,    dest="R3", src1="R1", src2="R4"),

Instruction("SW  R3,4(R2)",   InstType.STORE,  src1="R2", src2="R3", imm=0x2004),

Instruction("BEQ R3,R5,loop", InstType.BRANCH, src1="R3", src2="R5", imm=0x1020),

Instruction("ADDI R6,R0,1",   InstType.ALU,    dest="R6", src1="R0", imm=1),

Instruction("LW  R7,8(R2)",   InstType.LOAD,   dest="R7", src1="R2", imm=0x2008),

]

return instructions

if __name__ == "__main__":

print("\n  IMP-1 Industrial Monitoring Processor — Section 2")

print("  Computer Architecture CSA1203 | CO3\n")

instructions = build_sensor_program()

pipe = Pipeline5Stage()

pipe.run(instructions)

pipe.print_timing_diagram(instructions)

hazard_analysis_report(pipe)

execution_technique_comparison()

smt_analysis()

2)

"""

Section 2: Pipelined Processor Organization & Hazard Analysis

CSA1203 – Computer Architecture | IMP-1 Industrial Monitoring Processor

CO Mapping: CO3 | Marks: 30

Implements:

- 5-stage in-order pipeline (IF / ID / EX / MEM / WB)

- Pipeline registers (IF/ID, ID/EX, EX/MEM, MEM/WB)

- RAW hazard detection & forwarding unit

- Load-use stall insertion

- 2-bit saturating counter branch predictor (BHT)

- Structural hazard analysis

- Timing diagram generator

- Superscalar / OoO / SMT comparison

"""

from dataclasses import dataclass, field

from typing import Optional, List, Dict, Tuple

from enum import Enum, auto

# ─────────────────────────────────────────────────────────────

# 2.0  Instruction representation

# ─────────────────────────────────────────────────────────────

class InstType(Enum):

ALU    = "ALU"

LOAD   = "LOAD"

STORE  = "STORE"

BRANCH = "BRANCH"

JUMP   = "JUMP"

IO     = "IO_READ"

MUL    = "MULTIPLY"

NOP    = "NOP"

@dataclass

class Instruction:

label: str                  # human-readable name, e.g. "LW R1,0(R2)"

itype: InstType

dest:  Optional[str] = None # destination register

src1:  Optional[str] = None

src2:  Optional[str] = None

imm:   int = 0              # immediate / offset

def __str__(self):

return self.label

# ─────────────────────────────────────────────────────────────

# 2.1  Pipeline Registers

# ─────────────────────────────────────────────────────────────

@dataclass

class IFIDReg:

inst:  Optional[Instruction] = None

pc:    int = 0

@dataclass

class IDEXReg:

inst:      Optional[Instruction] = None

rs1_val:   int = 0

rs2_val:   int = 0

rd:        Optional[str] = None

imm:       int = 0

alu_src:   bool = False

mem_read:  bool = False

mem_write: bool = False

reg_write: bool = False

is_branch: bool = False

@dataclass

class EXMEMReg:

inst:      Optional[Instruction] = None

alu_result: int = 0

rd:        Optional[str] = None

rs2_val:   int = 0

mem_read:  bool = False

mem_write: bool = False

reg_write: bool = False

branch_taken: bool = False

branch_target: int = 0

@dataclass

class MEMWBReg:

inst:       Optional[Instruction] = None

rd:         Optional[str] = None

result:     int = 0         # ALU result or loaded memory data

reg_write:  bool = False

# ─────────────────────────────────────────────────────────────

# 2.2  2-bit Saturating Counter Branch Predictor (BHT)

# ─────────────────────────────────────────────────────────────

class BHTPredictor:

"""

512-entry Branch History Table with 2-bit saturating counters.

States: 0=Strongly NT, 1=Weakly NT, 2=Weakly T, 3=Strongly T

"""

STRONGLY_NT = 0

WEAKLY_NT   = 1

WEAKLY_T    = 2

STRONGLY_T  = 3

def __init__(self, entries: int = 512):

self.table = [self.WEAKLY_NT] * entries   # start neutral

self.hits  = 0

self.total = 0

def _index(self, pc: int) -> int:

return (pc >> 2) % len(self.table)

def predict(self, pc: int) -> bool:

"""Returns True (predict Taken) if counter ≥ 2."""

return self.table[self._index(pc)] >= self.WEAKLY_T

def update(self, pc: int, actually_taken: bool):

idx = self._index(pc)

old = self.table[idx]

if actually_taken:

self.table[idx] = min(self.STRONGLY_T, old + 1)

else:

self.table[idx] = max(self.STRONGLY_NT, old - 1)

predicted = (old >= self.WEAKLY_T)

self.total += 1

if predicted == actually_taken:

self.hits += 1

@property

def accuracy(self) -> float:

return self.hits / self.total if self.total else 0.0

def state_name(self, pc: int) -> str:

names = {0:"Strongly NT", 1:"Weakly NT", 2:"Weakly T", 3:"Strongly T"}

return names[self.table[self._index(pc)]]

# ─────────────────────────────────────────────────────────────

# 2.3  Register File

# ─────────────────────────────────────────────────────────────

class RegisterFile:

def __init__(self):

self.regs: Dict[str, int] = {f"R{i}": 0 for i in range(32)}

self.regs["R0"] = 0  # hardwired

def read(self, reg: str) -> int:

if reg is None:

return 0

return self.regs.get(reg, 0)

def write(self, reg: str, val: int):

if reg and reg != "R0":

self.regs[reg] = val & 0xFFFFFFFF   # 32-bit mask

# ─────────────────────────────────────────────────────────────

# 2.4  Simple Data Memory

# ─────────────────────────────────────────────────────────────

class DataMemory:

def __init__(self):

self.mem: Dict[int, int] = {}

# Pre-load sensor data at 0x2000+

for i in range(32):

self.mem[0x2000 + i*4] = 0x100 + i    # simulated sensor values

def load(self, addr: int) -> int:

return self.mem.get(addr & ~0x3, 0)        # word-aligned

def store(self, addr: int, val: int):

self.mem[addr & ~0x3] = val & 0xFFFFFFFF

# ─────────────────────────────────────────────────────────────

# 2.5  5-Stage Pipeline Simulator

# ─────────────────────────────────────────────────────────────

class Pipeline5Stage:

"""

Simulates the IMP-1 5-stage in-order pipeline.

Tracks: stalls, forwards, mispredictions, timing diagrams.

"""

def __init__(self):

self.rf  = RegisterFile()

self.mem = DataMemory()

self.bht = BHTPredictor(512)

# Pipeline registers

self.if_id  = IFIDReg()

self.id_ex  = IDEXReg()

self.ex_mem = EXMEMReg()

self.mem_wb = MEMWBReg()

# Counters

self.cycle        = 0

self.stalls       = 0

self.forwards_ex  = 0

self.forwards_mem = 0

self.flushes      = 0

# Timeline recording for diagram: {inst_label: {stage: cycle}}

self.timeline: Dict[str, Dict[str, int]] = {}

# ── helpers ────────────────────────────────────────────────

def _record(self, stage: str, inst: Optional[Instruction]):

if inst and inst.itype != InstType.NOP:

if inst.label not in self.timeline:

self.timeline[inst.label] = {}

self.timeline[inst.label][stage] = self.cycle

def _is_load_use_hazard(self) -> bool:

"""

Detect load-use hazard:

ID/EX register has a LOAD that writes to a register

that the next instruction (currently in IF/ID) reads.

"""

if not self.id_ex.mem_read:

return False

incoming = self.if_id.inst

if incoming is None or incoming.itype == InstType.NOP:

return False

rd = self.id_ex.rd

return rd in (incoming.src1, incoming.src2)

def _forwarding(self, rs: Optional[str]) -> Tuple[int, str]:

"""

Return forwarded value and source label for register rs.

Priority: EX/MEM > MEM/WB > Register File.

"""

if rs is None:

return 0, "RF"

if self.ex_mem.reg_write and self.ex_mem.rd == rs:

self.forwards_ex += 1

return self.ex_mem.alu_result, "EX/MEM"

if self.mem_wb.reg_write and self.mem_wb.rd == rs:

self.forwards_mem += 1

return self.mem_wb.result, "MEM/WB"

return self.rf.read(rs), "RF"

def _alu(self, itype: InstType, a: int, b: int) -> int:

if itype in (InstType.ALU, InstType.LOAD, InstType.STORE):

return (a + b) & 0xFFFFFFFF

if itype == InstType.MUL:

return (a * b) & 0xFFFFFFFF

if itype == InstType.BRANCH:

return a - b                            # compare for equality

return 0

# ── execute one clock cycle ─────────────────────────────────

def tick(self, next_inst: Optional[Instruction], pc: int):

self.cycle += 1

stall_this_cycle = False

# ── WB ──────────────────────────────────────────────────

if self.mem_wb.reg_write and self.mem_wb.rd:

self.rf.write(self.mem_wb.rd, self.mem_wb.result)

self._record("WB", self.mem_wb.inst)

# ── MEM ─────────────────────────────────────────────────

new_mem_wb = MEMWBReg(

inst      = self.ex_mem.inst,

rd        = self.ex_mem.rd,

reg_write = self.ex_mem.reg_write

)

if self.ex_mem.mem_read:

new_mem_wb.result = self.mem.load(self.ex_mem.alu_result)

elif self.ex_mem.mem_write:

self.mem.store(self.ex_mem.alu_result, self.ex_mem.rs2_val)

new_mem_wb.result = 0

else:

new_mem_wb.result = self.ex_mem.alu_result

self._record("MEM", self.ex_mem.inst)

# ── EX ──────────────────────────────────────────────────

a, src_a = self._forwarding(self.id_ex.inst.src1 if self.id_ex.inst else None)

if self.id_ex.alu_src:

b, src_b = self.id_ex.imm, "IMM"

else:

b, src_b = self._forwarding(self.id_ex.inst.src2 if self.id_ex.inst else None)

alu_out = self._alu(

self.id_ex.inst.itype if self.id_ex.inst else InstType.NOP,

a, b

)

# Branch resolution

branch_taken   = False

branch_target  = 0

if self.id_ex.is_branch:

branch_taken  = (alu_out == 0)           # BEQ: rs1==rs2

branch_target = self.id_ex.imm            # pre-computed target

predicted     = self.bht.predict(self.if_id.pc)

if predicted != branch_taken:

self.flushes += 1                     # flush 2 instructions

self.bht.update(self.if_id.pc, branch_taken)

new_ex_mem = EXMEMReg(

inst          = self.id_ex.inst,

alu_result    = alu_out,

rd            = self.id_ex.rd,

rs2_val       = b,

mem_read      = self.id_ex.mem_read,

mem_write     = self.id_ex.mem_write,

reg_write     = self.id_ex.reg_write,

branch_taken  = branch_taken,

branch_target = branch_target

)

self._record("EX", self.id_ex.inst)

# ── ID ──────────────────────────────────────────────────

# Load-use hazard check

stall_this_cycle = self._is_load_use_hazard()

if stall_this_cycle:

self.stalls += 1

new_id_ex = IDEXReg()           # bubble

else:

inst = self.if_id.inst

if inst is None:

new_id_ex = IDEXReg()

else:

new_id_ex = IDEXReg(

inst      = inst,

rs1_val   = self.rf.read(inst.src1),

rs2_val   = self.rf.read(inst.src2),

rd        = inst.dest,

imm       = inst.imm,

alu_src   = inst.itype in (InstType.LOAD, InstType.STORE, InstType.IO),

mem_read  = inst.itype == InstType.LOAD,

mem_write = inst.itype == InstType.STORE,

reg_write = inst.itype in (InstType.ALU, InstType.LOAD,

InstType.IO,  InstType.MUL),

is_branch = inst.itype == InstType.BRANCH

)

self._record("ID", self.if_id.inst if not stall_this_cycle else None)

# ── IF ──────────────────────────────────────────────────

if stall_this_cycle:

new_if_id = self.if_id         # freeze IF/ID

else:

new_if_id = IFIDReg(inst=next_inst, pc=pc)

self._record("IF", next_inst if not stall_this_cycle else None)

# Commit pipeline registers

self.if_id  = new_if_id

self.id_ex  = new_id_ex

self.ex_mem = new_ex_mem

self.mem_wb = new_mem_wb

return stall_this_cycle

# ── run a full instruction sequence ────────────────────────

def run(self, instructions: List[Instruction]):

"""

Drive the pipeline with a list of instructions.

Pads with NOPs to drain the pipeline at the end.

"""

NOP = Instruction("NOP", InstType.NOP)

program = instructions + [NOP] * 5    # drain pipeline

ip = 0                                # instruction pointer

pc = 0x1000

while ip < len(program):

fetch_inst = program[ip] if ip < len(program) else NOP

stalled = self.tick(fetch_inst, pc)

if not stalled:

ip += 1

pc += 4

# ── timing diagram ─────────────────────────────────────────

def print_timing_diagram(self, instructions: List[Instruction]):

stage_map = {"IF": "IF", "ID": "ID", "EX": "EX", "MEM": "MEM", "WB": "WB"}

max_cycle = max(

(max(v.values()) for v in self.timeline.values() if v),

default=0

)

print("\n" + "="*80)

print("  PIPELINE TIMING DIAGRAM")

print("="*80)

header = f"  {'Instruction':<30}" + "".join(f" C{c:<3}" for c in range(1, max_cycle+2))

print(header)

print("  " + "-"*75)

for inst in instructions:

lbl = inst.label

stages = self.timeline.get(lbl, {})

row = f"  {lbl:<30}"

for c in range(1, max_cycle+2):

cell = "    "

for stage, cyc in stages.items():

if cyc == c:

abbr = stage[:2] if stage == "MEM" else stage

cell = f" {stage:<3}"

row += cell

print(row)

print()

# ─────────────────────────────────────────────────────────────

# 2.6  Hazard Analysis Report

# ─────────────────────────────────────────────────────────────

def hazard_analysis_report(pipeline: Pipeline5Stage):

print("\n" + "="*70)

print("  HAZARD ANALYSIS REPORT")

print("="*70)

total_instructions = sum(

1 for lbl, stages in pipeline.timeline.items()

if stages

)

print(f"\n  Total instructions executed : {total_instructions}")

print(f"  Load-use stalls inserted    : {pipeline.stalls}")

print(f"  Branch mispredictions       : {pipeline.flushes}")

print(f"  EX/MEM forwards applied     : {pipeline.forwards_ex}")

print(f"  MEM/WB forwards applied     : {pipeline.forwards_mem}")

print(f"  Total cycles elapsed        : {pipeline.cycle}")

cpi = pipeline.cycle / total_instructions if total_instructions else 0

print(f"  Effective CPI               : {cpi:.3f}")

print(f"  BHT accuracy                : {pipeline.bht.accuracy*100:.1f}%")

print("\n  Data Hazard Summary (RAW):")

print("  ┌──────────────────────────────────────────────┬────────────────────┐")

print("  │ Hazard Type                                  │ Resolution         │")

print("  ├──────────────────────────────────────────────┼────────────────────┤")

print("  │ ALU→ALU (EX produces, next EX reads)         │ EX/MEM forwarding  │")

print("  │ Load-Use (LOAD EX produces, next EX reads)   │ 1-cycle stall+fwd  │")

print("  │ ALU→ALU 2nd behind (MEM produces, EX reads) │ MEM/WB forwarding  │")

print("  │ WAW / WAR (write-after-write/read)           │ In-order prevents  │")

print("  └──────────────────────────────────────────────┴────────────────────┘")

print("\n  Control Hazard Summary (Branch):")

print("  2-bit saturating counter BHT (512 entries)")

print("  Misprediction penalty = 2 cycles (flush 2 fetched instructions)")

print(f"  Accuracy = {pipeline.bht.accuracy*100:.1f}% | Flushes = {pipeline.flushes}")

print("\n  Structural Hazard Summary:")

print("  IF vs MEM memory conflict → Resolved by Harvard split L1-I / L1-D")

print("  Register file WB+ID same cycle → Dual-port RF (write-first convention)")

print("  MUL multi-cycle blocks EX → Separate 4-stage multiply pipeline")

# ─────────────────────────────────────────────────────────────

# 2.7  Execution Technique Comparison

# ─────────────────────────────────────────────────────────────

def execution_technique_comparison():

techniques = [

{

"name":       "In-order Scalar (IMP-1 baseline)",

"mechanism":  "1 inst/cycle; stall on hazard",

"ipc":        "≤1.0  (~0.72 effective)",

"hw_cost":    "Low",

"power":      "Low",

"rt_suit":    "Excellent – deterministic latency",

"recommended": True

},

{

"name":       "In-order Superscalar 2-wide",

"mechanism":  "2 insts/cycle from aligned pairs",

"ipc":        "≤2.0  (~1.4 effective)",

"hw_cost":    "Medium",

"power":      "Medium",

"rt_suit":    "Good – minor alignment variance",

"recommended": True

},

{

"name":       "Out-of-Order (OoO)",

"mechanism":  "ROB + reservation stations; dynamic schedule",

"ipc":        "≤4–6 (~3.0 effective)",

"hw_cost":    "Very High",

"power":      "High",

"rt_suit":    "POOR – non-deterministic commit order",

"recommended": False

},

{

"name":       "Speculative Execution",

"mechanism":  "Execute past unresolved branches; rollback",

"ipc":        "≤4+  (~3.5 effective)",

"hw_cost":    "Very High",

"power":      "Very High",

"rt_suit":    "POOR – Spectre-class security risk",

"recommended": False

},

{

"name":       "SMT 2-thread",

"mechanism":  "Two HW threads share pipeline",

"ipc":        "≤2.0 (~1.8 effective)",

"hw_cost":    "Medium-High",

"power":      "Medium",

"rt_suit":    "Good – I/O+compute thread separation",

"recommended": True

},

]

print("\n" + "="*90)

print("  EXECUTION TECHNIQUE COMPARISON (Section 2.4)")

print("="*90)

fmt = "  {:<30} {:^14} {:^12} {:^10} {:^14} {:^6}"

print(fmt.format("Technique", "IPC potential", "HW cost", "Power", "RT Suitable", "Pick?"))

print("  " + "-"*85)

for t in techniques:

pick = "✓" if t["recommended"] else "✗"

print(fmt.format(

t["name"][:30], t["ipc"], t["hw_cost"], t["power"], t["rt_suit"][:14], pick

))

print("\n  Recommendation: In-order pipeline + 2-thread SMT (sensor I/O + compute)")

print("  OoO and Speculative execution REJECTED – violate real-time determinism.")

# ─────────────────────────────────────────────────────────────

# 2.8  SMT Analysis

# ─────────────────────────────────────────────────────────────

def smt_analysis():

thread0_stall_rate = 0.13          # sensor I/O thread stalls 13% of cycles

thread1_compute    = 1 - 0.02     # compute thread stalls only 2%

baseline_util      = 1 - thread0_stall_rate

smt_util           = baseline_util + thread0_stall_rate * thread1_compute

print("\n" + "="*70)

print("  SMT ANALYSIS (Section 2.5)")

print("="*70)

print(f"\n  Thread 0 (Sensor I/O)  : stalls {thread0_stall_rate*100:.0f}% of cycles (DMA/memory waits)")

print(f"  Thread 1 (Processing)  : stalls  ~2% of cycles (CPU-bound)")

print(f"\n  Baseline utilisation   : {baseline_util*100:.0f}%")

print(f"  SMT utilisation        : {smt_util*100:.0f}%  (+{(smt_util-baseline_util)*100:.0f}pp)")

ipc_gain = smt_util / baseline_util

print(f"  Estimated IPC gain     : {ipc_gain:.2f}×")

print(f"\n  Verdict: SMT SUITABLE — natural workload decomposition (I/O + compute)")

# ─────────────────────────────────────────────────────────────

# Main

# ─────────────────────────────────────────────────────────────

def build_sensor_program() -> List[Instruction]:

"""

Sensor monitoring inner loop (6 key instructions from the report):

I1: LW R1, 0(R2)        ← load sensor reading

I2: ADD R3, R1, R4      ← accumulate (RAW hazard on R1 → load-use stall)

I3: SW R3, 4(R2)        ← store result

I4: BEQ R3, R5, loop    ← compare against threshold → branch

I5: ADDI R6, R0, 1      ← set alarm flag

I6: LW R7, 8(R2)        ← load next sensor register

"""

R = lambda name, default=0: name  # register name helper

instructions = [

Instruction("LW  R1,0(R2)",   InstType.LOAD,   dest="R1", src1="R2", imm=0x2000),

Instruction("ADD R3,R1,R4",   InstType.ALU,    dest="R3", src1="R1", src2="R4"),

Instruction("SW  R3,4(R2)",   InstType.STORE,  src1="R2", src2="R3", imm=0x2004),

Instruction("BEQ R3,R5,loop", InstType.BRANCH, src1="R3", src2="R5", imm=0x1020),

Instruction("ADDI R6,R0,1",   InstType.ALU,    dest="R6", src1="R0", imm=1),

Instruction("LW  R7,8(R2)",   InstType.LOAD,   dest="R7", src1="R2", imm=0x2008),

]

return instructions

if __name__ == "__main__":

print("\n  IMP-1 Industrial Monitoring Processor — Section 2")

print("  Computer Architecture CSA1203 | CO3\n")

instructions = build_sensor_program()

pipe = Pipeline5Stage()

pipe.run(instructions)

pipe.print_timing_diagram(instructions)

hazard_analysis_report(pipe)

execution_technique_comparison()

smt_analysis()

3)

"""

Section 3: Hierarchical Memory System & Performance Evaluation

CSA1203 – Computer Architecture | IMP-1 Industrial Monitoring Processor

CO Mapping: CO4 | Marks: 25

Implements:

- 3-level cache hierarchy simulation (L1-I, L1-D, L2, L3)

- LRU / Pseudo-LRU replacement policy

- AMAT calculation (step-by-step and sensitivity analysis)

- Virtual memory with paging (32-bit, 4KB pages)

- TLB simulation (64-entry fully-associative, LRU)

- EMAT calculation

- Memory technology comparison table

"""

import random

from collections import OrderedDict

from dataclasses import dataclass

from typing import Dict, List, Optional, Tuple

# ─────────────────────────────────────────────────────────────

# 3.1  Cache Configuration

# ─────────────────────────────────────────────────────────────

@dataclass

class CacheConfig:

name:          str

capacity_kb:   int

associativity: int      # ways

block_size_b:  int      # bytes per block (64)

hit_latency:   int      # cycles

hit_rate:      float    # fraction of accesses that hit THIS level

write_policy:  str      # "write-back" | "write-through" | "read-only"

replacement:   str      # "LRU" | "Pseudo-LRU"

power_mw:      int

@property

def num_sets(self) -> int:

return (self.capacity_kb * 1024) // (self.associativity * self.block_size_b)

@property

def miss_rate(self) -> float:

return 1.0 - self.hit_rate

def __str__(self):

return (f"{self.name}: {self.capacity_kb} KB, "

f"{self.associativity}-way SA, "

f"{self.num_sets} sets, "

f"{self.hit_latency}-cycle hit, "

f"{self.hit_rate*100:.0f}% hit-rate")

# IMP-1 cache hierarchy definitions

L1I = CacheConfig("L1-I", 32,  4,  64,  2,  0.97, "read-only",  "LRU",        45)

L1D = CacheConfig("L1-D", 32,  4,  64,  2,  0.92, "write-back", "LRU",        45)

L2  = CacheConfig("L2",   256, 8,  64,  8,  0.97, "write-back", "Pseudo-LRU", 120)

L3  = CacheConfig("L3",  2048, 16, 64, 30,  0.99, "write-back", "Pseudo-LRU", 400)

DRAM_LATENCY = 200   # cycles

# ─────────────────────────────────────────────────────────────

# 3.2  LRU Cache Simulator (set-associative)

# ─────────────────────────────────────────────────────────────

class LRUCache:

"""

General-purpose set-associative LRU cache.

Each set is an OrderedDict: {tag: data} (MRU at right end).

"""

def __init__(self, cfg: CacheConfig):

self.cfg      = cfg

self.sets: List[OrderedDict] = [OrderedDict() for _ in range(cfg.num_sets)]

self.hits     = 0

self.misses   = 0

self.evictions = 0

def _parse_addr(self, addr: int) -> Tuple[int, int, int]:

"""Split address into (tag, set_index, block_offset)."""

offset_bits = int(self.cfg.block_size_b).bit_length() - 1

index_bits  = int(self.cfg.num_sets).bit_length() - 1

offset      = addr & ((1 << offset_bits) - 1)

index       = (addr >> offset_bits) & ((1 << index_bits) - 1)

tag         = addr >> (offset_bits + index_bits)

return tag, index, offset

def access(self, addr: int, data: int = 0, write: bool = False) -> Tuple[bool, int]:

"""

Perform a cache access.

Returns (hit: bool, latency: int).

"""

tag, idx, _ = self._parse_addr(addr)

s = self.sets[idx]

if tag in s:                     # HIT

self.hits += 1

s.move_to_end(tag)           # update LRU order

return True, self.cfg.hit_latency

# MISS

self.misses += 1

if len(s) >= self.cfg.associativity:

s.popitem(last=False)        # evict LRU

self.evictions += 1

s[tag] = data                    # install block

return False, self.cfg.hit_latency

@property

def hit_rate_observed(self) -> float:

total = self.hits + self.misses

return self.hits / total if total else 0.0

def stats(self) -> str:

total = self.hits + self.misses

return (f"{self.cfg.name}: {total} accesses | "

f"{self.hits} hits ({self.hit_rate_observed*100:.1f}%) | "

f"{self.misses} misses | {self.evictions} evictions")

# ─────────────────────────────────────────────────────────────

# 3.3  AMAT Calculator

# ─────────────────────────────────────────────────────────────

class AMATCalculator:

"""

Computes Average Memory Access Time analytically using the

recursive inclusive formula:

AMAT = h1*t1 + (1-h1)*[h2*t2 + (1-h2)*[h3*t3 + (1-h3)*t_DRAM]]

"""

def __init__(self,

h_l1=0.92, t_l1=2,

h_l2=0.97, t_l2=8,

h_l3=0.99, t_l3=30,

t_dram=200):

self.h_l1, self.t_l1 = h_l1, t_l1

self.h_l2, self.t_l2 = h_l2, t_l2

self.h_l3, self.t_l3 = h_l3, t_l3

self.t_dram          = t_dram

def compute(self) -> Dict:

# Step 1: L3 miss penalty

l3_miss = self.t_l3 + (1 - self.h_l3) * self.t_dram

# Step 2: L2 miss penalty (conditional miss → L3 or DRAM)

l2_miss = self.t_l2 + (1 - self.h_l2) * l3_miss

# Step 3: AMAT (from L1 perspective)

amat    = self.t_l1 + (1 - self.h_l1) * l2_miss

return {

"l3_miss_penalty": l3_miss,

"l2_miss_penalty": l2_miss,

"amat_cycles":     amat,

"amat_ns":         amat * 2,          # at 500 MHz, 1 cycle = 2 ns

}

def sensitivity(self) -> List[Dict]:

"""Vary L1 hit rate and report AMAT impact."""

rows = []

for h1 in [0.85, 0.90, 0.92, 0.95, 0.98]:

calc = AMATCalculator(h_l1=h1, t_l1=self.t_l1,

h_l2=self.h_l2, t_l2=self.t_l2,

h_l3=self.h_l3, t_l3=self.t_l3,

t_dram=self.t_dram)

r = calc.compute()

rows.append({

"h_l1": h1,

"amat_cycles": r["amat_cycles"],

"amat_ns":     r["amat_ns"],

"note": "baseline" if h1 == self.h_l1 else ""

})

return rows

def report(self):

r = self.compute()

print("\n" + "="*70)

print("  AMAT STEP-BY-STEP CALCULATION (Section 3.2)")

print("="*70)

print(f"\n  Given:")

print(f"    L1-D: hit-rate={self.h_l1*100:.0f}%  latency={self.t_l1} cycles")

print(f"    L2  : hit-rate={self.h_l2*100:.0f}%  latency={self.t_l2} cycles  (of L1 misses)")

print(f"    L3  : hit-rate={self.h_l3*100:.0f}%  latency={self.t_l3} cycles  (of L2 misses)")

print(f"    DRAM: latency={self.t_dram} cycles")

print(f"\n  Step 1 — L3 miss penalty:")

print(f"    = t_L3 + (1 − h_L3) × t_DRAM")

print(f"    = {self.t_l3} + (1 − {self.h_l3}) × {self.t_dram}")

print(f"    = {self.t_l3} + {(1-self.h_l3)*self.t_dram:.2f}")

print(f"    = {r['l3_miss_penalty']:.2f} cycles")

print(f"\n  Step 2 — L2 miss penalty:")

print(f"    = t_L2 + (1 − h_L2) × L3_miss_penalty")

print(f"    = {self.t_l2} + (1 − {self.h_l2}) × {r['l3_miss_penalty']:.2f}")

print(f"    = {self.t_l2} + {(1-self.h_l2)*r['l3_miss_penalty']:.4f}")

print(f"    = {r['l2_miss_penalty']:.4f} cycles")

print(f"\n  Step 3 — AMAT:")

print(f"    = t_L1 + (1 − h_L1) × L2_miss_penalty")

print(f"    = {self.t_l1} + (1 − {self.h_l1}) × {r['l2_miss_penalty']:.4f}")

print(f"    = {self.t_l1} + {(1-self.h_l1)*r['l2_miss_penalty']:.4f}")

print(f"    ≈ {r['amat_cycles']:.3f} cycles  ≈ {r['amat_ns']:.2f} ns  (at 500 MHz)")

print("\n  AMAT Sensitivity (varying L1 hit rate):")

print(f"  {'L1 Hit Rate':>12} {'AMAT (cycles)':>15} {'AMAT (ns)':>12} {'Note':>10}")

print("  " + "-"*50)

for row in self.sensitivity():

print(f"  {row['h_l1']*100:>11.0f}% {row['amat_cycles']:>15.3f} "

f"{row['amat_ns']:>12.2f} {row['note']:>10}")

# ─────────────────────────────────────────────────────────────

# 3.4  TLB Simulation (64-entry fully-associative LRU)

# ─────────────────────────────────────────────────────────────

class TLB:

"""

64-entry fully-associative TLB with LRU replacement.

Maps Virtual Page Number (VPN) → Physical Frame Number (PFN).

"""

PAGE_SIZE   = 4096          # 4 KB

ENTRIES     = 64

TLB_LATENCY = 1             # cycles

def __init__(self):

self.table: OrderedDict = OrderedDict()   # {vpn: pfn}

self.hits   = 0

self.misses = 0

# Pre-warm TLB with common sensor buffer pages

for vpn in range(0x200, 0x210):

pfn = vpn + 0x800

self.table[vpn] = pfn

def _vpn(self, va: int) -> int:

return va >> 12       # upper 20 bits

def _offset(self, va: int) -> int:

return va & 0xFFF     # lower 12 bits

def translate(self, va: int, page_table: Dict[int, int]) -> Tuple[int, int, bool]:

"""

Translate virtual address to physical address.

Returns (physical_addr, latency_cycles, tlb_hit).

"""

vpn = self._vpn(va)

if vpn in self.table:            # TLB HIT

pfn = self.table[vpn]

self.table.move_to_end(vpn)

pa = (pfn << 12) | self._offset(va)

self.hits += 1

return pa, self.TLB_LATENCY, True

# TLB MISS — page table walk (5–10 cycles if PT cached in L2)

self.misses += 1

pfn = page_table.get(vpn, vpn + 0x500)    # default mapping

if len(self.table) >= self.ENTRIES:

self.table.popitem(last=False)          # evict LRU

self.table[vpn] = pfn

pa = (pfn << 12) | self._offset(va)

return pa, 8, False              # TLB miss = 8 cycles (PT walk in L2)

@property

def hit_rate(self) -> float:

total = self.hits + self.misses

return self.hits / total if total else 0.0

class VirtualMemorySystem:

"""

32-bit virtual address space: 20-bit VPN + 12-bit offset.

Physical space: 32-bit (up to 4 GB).

Page table: 2^20 PTEs × 4 B = 4 MB.

"""

def __init__(self, amat_calc: AMATCalculator):

self.amat_calc   = amat_calc

self.tlb         = TLB()

self.page_table  = {}         # VPN → PFN mapping

self._init_page_table()

def _init_page_table(self):

"""Simulate a page table for the sensor buffer region."""

for vpn in range(0, 0x1000):

self.page_table[vpn] = vpn + 0x100    # simple offset mapping

def address_translation_trace(self, va: int):

"""

Demonstrate step-by-step address translation for a given VA.

"""

vpn    = va >> 12

offset = va & 0xFFF

pa, latency, hit = self.tlb.translate(va, self.page_table)

pfn = pa >> 12

print("\n" + "="*70)

print(f"  ADDRESS TRANSLATION TRACE  (VA = 0x{va:08X})")

print("="*70)

steps = [

("1. Extract VPN", f"VA[31:12] = VPN", f"VPN = 0x{vpn:05X}"),

("2. Extract Offset", f"VA[11:0]  = Offset", f"Offset = 0x{offset:03X}"),

("3. TLB Lookup", f"Search TLB for VPN = 0x{vpn:05X}",

f"{'TLB HIT' if hit else 'TLB MISS (page table walk)'}  → PFN = 0x{pfn:05X}"),

("4. Form PA", f"PA = PFN[31:12] | Offset",

f"PA = 0x{pa:08X}"),

("5. Cache Lookup", f"Use PA to index L1-D cache",

f"L1-D access → latency = {latency} (TLB) + 2 (L1) = {latency+2} cycles"),

]

for label, op, result in steps:

print(f"  {label:<20} : {op:<40} → {result}")

def emat(self, tlb_hit_rate=0.985) -> float:

"""

EMAT = h_TLB × (t_TLB + AMAT) + (1 − h_TLB) × (t_PTwalk + AMAT)

"""

amat_r = self.amat_calc.compute()

amat   = amat_r["amat_cycles"]

t_tlb  = TLB.TLB_LATENCY

t_pt   = 8        # page-table walk (L2-cached)

emat   = tlb_hit_rate * (t_tlb + amat) + (1 - tlb_hit_rate) * (t_pt + amat)

return emat

def emat_report(self, tlb_hit_rate=0.985):

amat   = self.amat_calc.compute()["amat_cycles"]

e      = self.emat(tlb_hit_rate)

t_tlb  = TLB.TLB_LATENCY

t_pt   = 8

print("\n" + "="*70)

print("  EFFECTIVE MEMORY ACCESS TIME WITH TLB (Section 3.3.3)")

print("="*70)

print(f"\n  TLB hit rate    = {tlb_hit_rate*100:.1f}%")

print(f"  TLB hit latency = {t_tlb} cycle")

print(f"  TLB miss penalty= {t_pt} cycles  (page-table walk, L2-cached)")

print(f"  AMAT            = {amat:.3f} cycles")

print(f"\n  EMAT = h_TLB × (t_TLB + AMAT) + (1−h_TLB) × (t_PT + AMAT)")

print(f"       = {tlb_hit_rate} × ({t_tlb} + {amat:.3f})")

print(f"         + {1-tlb_hit_rate} × ({t_pt} + {amat:.3f})")

term1 = tlb_hit_rate * (t_tlb + amat)

term2 = (1 - tlb_hit_rate) * (t_pt + amat)

print(f"       = {term1:.2f} + {term2:.2f}")

print(f"       = {e:.2f} cycles")

# ─────────────────────────────────────────────────────────────

# 3.5  Cache Hierarchy Simulation (access trace)

# ─────────────────────────────────────────────────────────────

def run_cache_simulation():

"""

Simulate sensor-loop memory accesses through the L1/L2/L3 hierarchy.

Access pattern: sequential sensor array traversal with high temporal locality.

"""

l1d = LRUCache(L1D)

l2  = LRUCache(L2)

l3  = LRUCache(L3)

# Simulate 1000 memory accesses: sensor array read, then repeated access

addresses = []

base = 0x2000

# First pass: sequential (cold misses)

for i in range(64):

addresses.append(base + i * 4)

# Second pass: same addresses (hot hits)

for i in range(64):

addresses.append(base + i * 4)

# Third pass: partial — 80% from hot set, 20% new

for i in range(64):

if i % 5 == 0:

addresses.append(base + 256 + i * 4)  # cold

else:

addresses.append(base + i * 4)        # hot

total_latency = 0

for addr in addresses:

val = addr & 0xFF

hit1, lat1 = l1d.access(addr, val)

if not hit1:

hit2, lat2 = l2.access(addr, val)

if not hit2:

hit3, lat3 = l3.access(addr, val)

if not hit3:

total_latency += DRAM_LATENCY

total_latency += lat3

total_latency += lat2

total_latency += lat1

print("\n" + "="*70)

print("  CACHE HIERARCHY SIMULATION RESULTS (Section 3.1)")

print("="*70)

print(f"\n  Total accesses  : {len(addresses)}")

print(f"  Total latency   : {total_latency} cycles")

print(f"  Avg per access  : {total_latency/len(addresses):.2f} cycles\n")

for cache in [l1d, l2, l3]:

print(f"  {cache.stats()}")

# ─────────────────────────────────────────────────────────────

# 3.6  Memory Technology Comparison

# ─────────────────────────────────────────────────────────────

def memory_technology_comparison():

technologies = [

{

"name":         "SRAM (6T)",

"read_lat":     "1–5 ns",

"write_lat":    "1–5 ns",

"endurance":    ">10¹⁵ cycles",

"bandwidth":    ">100 GB/s",

"nonvolatile":  "No",

"cost_gb":      "~$5,000",

"density":      "Low",

"standby_pwr":  "Low",

"imp1_role":    "L1, L2, L3 cache",

"maturity":     "Mature"

},

{

"name":         "DRAM (LPDDR4)",

"read_lat":     "40–60 ns",

"write_lat":    "40–60 ns",

"endurance":    ">10¹⁵ cycles",

"bandwidth":    "51.2 GB/s",

"nonvolatile":  "No",

"cost_gb":      "~$5",

"density":      "High",

"standby_pwr":  "Medium (refresh)",

"imp1_role":    "Main Memory (2 GB)",

"maturity":     "Mature"

},

{

"name":         "NAND Flash",

"read_lat":     "25–100 µs",

"write_lat":    "200–1500 µs",

"endurance":    "10³–10⁵ cycles",

"bandwidth":    "0.5–3 GB/s",

"nonvolatile":  "Yes",

"cost_gb":      "~$0.10",

"density":      "Very High",

"standby_pwr":  "Zero",

"imp1_role":    "Sensor event log (64 GB)",

"maturity":     "Mature"

},

{

"name":         "MRAM",

"read_lat":     "2–10 ns",

"write_lat":    "2–10 ns",

"endurance":    ">10¹² cycles",

"bandwidth":    ">10 GB/s",

"nonvolatile":  "Yes",

"cost_gb":      "~$50",

"density":      "Medium",

"standby_pwr":  "Zero",

"imp1_role":    "NV-cache (future, 8 MB)",

"maturity":     "Emerging production"

},

{

"name":         "RRAM",

"read_lat":     "10–50 ns",

"write_lat":    "10–50 ns",

"endurance":    "10⁶–10¹² cycles",

"bandwidth":    "1–10 GB/s",

"nonvolatile":  "Yes",

"cost_gb":      "~$10 (proj.)",

"density":      "High (emerging)",

"standby_pwr":  "Zero",

"imp1_role":    "Fast event buffer (future)",

"maturity":     "Early production"

},

]

print("\n" + "="*110)

print("  MEMORY TECHNOLOGY COMPARISON (Section 3.4)")

print("="*110)

cols = ("Technology", "Read Lat.", "NV?", "Endurance", "Cost/GB", "IMP-1 Role")

fmt  = "  {:<18} {:>12} {:>5} {:>16} {:>10} {}"

print(fmt.format(*cols))

print("  " + "-"*100)

for t in technologies:

print(fmt.format(

t["name"], t["read_lat"], t["nonvolatile"],

t["endurance"], t["cost_gb"], t["imp1_role"]

))

print("\n  Selection Rationale:")

print("  SRAM  : Only technology meeting 2–30 cycle cache latency at KB–MB scales.")

print("  LPDDR4: Lowest cost/GB; 0.6V core (50% power vs DDR4); 51.2 GB/s for DMA bursts.")

print("  NAND  : Non-volatile long-term sensor log; wear-leveling manages 10⁵ cycle limit.")

print("  MRAM  : Recommended future upgrade — near-SRAM speed + non-volatile (alarm state).")

print("  RRAM  : Too immature for safety-critical deployment — future roadmap item.")

# ─────────────────────────────────────────────────────────────

# Main

# ─────────────────────────────────────────────────────────────

if __name__ == "__main__":

print("\n  IMP-1 Industrial Monitoring Processor — Section 3")

print("  Computer Architecture CSA1203 | CO4\n")

# Print cache configuration

print("="*70)

print("  CACHE HIERARCHY CONFIGURATION")

print("="*70)

for cfg in [L1I, L1D, L2, L3]:

print(f"  {cfg}")

print(f"  DRAM: LPDDR4, 200-cycle latency, 51.2 GB/s")

# Run simulation

run_cache_simulation()

# AMAT calculation

amat_calc = AMATCalculator()

amat_calc.report()

# Virtual memory

vm = VirtualMemorySystem(amat_calc)

vm.address_translation_trace(0x00402010)    # sensor buffer VA from report

vm.emat_report(0.985)

# Memory technology comparison

memory_technology_comparison()

4)

"""

Section 3: Hierarchical Memory System & Performance Evaluation

CSA1203 – Computer Architecture | IMP-1 Industrial Monitoring Processor

CO Mapping: CO4 | Marks: 25

Implements:

- 3-level cache hierarchy simulation (L1-I, L1-D, L2, L3)

- LRU / Pseudo-LRU replacement policy

- AMAT calculation (step-by-step and sensitivity analysis)

- Virtual memory with paging (32-bit, 4KB pages)

- TLB simulation (64-entry fully-associative, LRU)

- EMAT calculation

- Memory technology comparison table

"""

import random

from collections import OrderedDict

from dataclasses import dataclass

from typing import Dict, List, Optional, Tuple

# ─────────────────────────────────────────────────────────────

# 3.1  Cache Configuration

# ─────────────────────────────────────────────────────────────

@dataclass

class CacheConfig:

name:          str

capacity_kb:   int

associativity: int      # ways

block_size_b:  int      # bytes per block (64)

hit_latency:   int      # cycles

hit_rate:      float    # fraction of accesses that hit THIS level

write_policy:  str      # "write-back" | "write-through" | "read-only"

replacement:   str      # "LRU" | "Pseudo-LRU"

power_mw:      int

@property

def num_sets(self) -> int:

return (self.capacity_kb * 1024) // (self.associativity * self.block_size_b)

@property

def miss_rate(self) -> float:

return 1.0 - self.hit_rate

def __str__(self):

return (f"{self.name}: {self.capacity_kb} KB, "

f"{self.associativity}-way SA, "

f"{self.num_sets} sets, "

f"{self.hit_latency}-cycle hit, "

f"{self.hit_rate*100:.0f}% hit-rate")

# IMP-1 cache hierarchy definitions

L1I = CacheConfig("L1-I", 32,  4,  64,  2,  0.97, "read-only",  "LRU",        45)

L1D = CacheConfig("L1-D", 32,  4,  64,  2,  0.92, "write-back", "LRU",        45)

L2  = CacheConfig("L2",   256, 8,  64,  8,  0.97, "write-back", "Pseudo-LRU", 120)

L3  = CacheConfig("L3",  2048, 16, 64, 30,  0.99, "write-back", "Pseudo-LRU", 400)

DRAM_LATENCY = 200   # cycles

# ─────────────────────────────────────────────────────────────

# 3.2  LRU Cache Simulator (set-associative)

# ─────────────────────────────────────────────────────────────

class LRUCache:

"""

General-purpose set-associative LRU cache.

Each set is an OrderedDict: {tag: data} (MRU at right end).

"""

def __init__(self, cfg: CacheConfig):

self.cfg      = cfg

self.sets: List[OrderedDict] = [OrderedDict() for _ in range(cfg.num_sets)]

self.hits     = 0

self.misses   = 0

self.evictions = 0

def _parse_addr(self, addr: int) -> Tuple[int, int, int]:

"""Split address into (tag, set_index, block_offset)."""

offset_bits = int(self.cfg.block_size_b).bit_length() - 1

index_bits  = int(self.cfg.num_sets).bit_length() - 1

offset      = addr & ((1 << offset_bits) - 1)

index       = (addr >> offset_bits) & ((1 << index_bits) - 1)

tag         = addr >> (offset_bits + index_bits)

return tag, index, offset

def access(self, addr: int, data: int = 0, write: bool = False) -> Tuple[bool, int]:

"""

Perform a cache access.

Returns (hit: bool, latency: int).

"""

tag, idx, _ = self._parse_addr(addr)

s = self.sets[idx]

if tag in s:                     # HIT

self.hits += 1

s.move_to_end(tag)           # update LRU order

return True, self.cfg.hit_latency

# MISS

self.misses += 1

if len(s) >= self.cfg.associativity:

s.popitem(last=False)        # evict LRU

self.evictions += 1

s[tag] = data                    # install block

return False, self.cfg.hit_latency

@property

def hit_rate_observed(self) -> float:

total = self.hits + self.misses

return self.hits / total if total else 0.0

def stats(self) -> str:

total = self.hits + self.misses

return (f"{self.cfg.name}: {total} accesses | "

f"{self.hits} hits ({self.hit_rate_observed*100:.1f}%) | "

f"{self.misses} misses | {self.evictions} evictions")

# ─────────────────────────────────────────────────────────────

# 3.3  AMAT Calculator

# ─────────────────────────────────────────────────────────────

class AMATCalculator:

"""

Computes Average Memory Access Time analytically using the

recursive inclusive formula:

AMAT = h1*t1 + (1-h1)*[h2*t2 + (1-h2)*[h3*t3 + (1-h3)*t_DRAM]]

"""

def __init__(self,

h_l1=0.92, t_l1=2,

h_l2=0.97, t_l2=8,

h_l3=0.99, t_l3=30,

t_dram=200):

self.h_l1, self.t_l1 = h_l1, t_l1

self.h_l2, self.t_l2 = h_l2, t_l2

self.h_l3, self.t_l3 = h_l3, t_l3

self.t_dram          = t_dram

def compute(self) -> Dict:

# Step 1: L3 miss penalty

l3_miss = self.t_l3 + (1 - self.h_l3) * self.t_dram

# Step 2: L2 miss penalty (conditional miss → L3 or DRAM)

l2_miss = self.t_l2 + (1 - self.h_l2) * l3_miss

# Step 3: AMAT (from L1 perspective)

amat    = self.t_l1 + (1 - self.h_l1) * l2_miss

return {

"l3_miss_penalty": l3_miss,

"l2_miss_penalty": l2_miss,

"amat_cycles":     amat,

"amat_ns":         amat * 2,          # at 500 MHz, 1 cycle = 2 ns

}

def sensitivity(self) -> List[Dict]:

"""Vary L1 hit rate and report AMAT impact."""

rows = []

for h1 in [0.85, 0.90, 0.92, 0.95, 0.98]:

calc = AMATCalculator(h_l1=h1, t_l1=self.t_l1,

h_l2=self.h_l2, t_l2=self.t_l2,

h_l3=self.h_l3, t_l3=self.t_l3,

t_dram=self.t_dram)

r = calc.compute()

rows.append({

"h_l1": h1,

"amat_cycles": r["amat_cycles"],

"amat_ns":     r["amat_ns"],

"note": "baseline" if h1 == self.h_l1 else ""

})

return rows

def report(self):

r = self.compute()

print("\n" + "="*70)

print("  AMAT STEP-BY-STEP CALCULATION (Section 3.2)")

print("="*70)

print(f"\n  Given:")

print(f"    L1-D: hit-rate={self.h_l1*100:.0f}%  latency={self.t_l1} cycles")

print(f"    L2  : hit-rate={self.h_l2*100:.0f}%  latency={self.t_l2} cycles  (of L1 misses)")

print(f"    L3  : hit-rate={self.h_l3*100:.0f}%  latency={self.t_l3} cycles  (of L2 misses)")

print(f"    DRAM: latency={self.t_dram} cycles")

print(f"\n  Step 1 — L3 miss penalty:")

print(f"    = t_L3 + (1 − h_L3) × t_DRAM")

print(f"    = {self.t_l3} + (1 − {self.h_l3}) × {self.t_dram}")

print(f"    = {self.t_l3} + {(1-self.h_l3)*self.t_dram:.2f}")

print(f"    = {r['l3_miss_penalty']:.2f} cycles")

print(f"\n  Step 2 — L2 miss penalty:")

print(f"    = t_L2 + (1 − h_L2) × L3_miss_penalty")

print(f"    = {self.t_l2} + (1 − {self.h_l2}) × {r['l3_miss_penalty']:.2f}")

print(f"    = {self.t_l2} + {(1-self.h_l2)*r['l3_miss_penalty']:.4f}")

print(f"    = {r['l2_miss_penalty']:.4f} cycles")

print(f"\n  Step 3 — AMAT:")

print(f"    = t_L1 + (1 − h_L1) × L2_miss_penalty")

print(f"    = {self.t_l1} + (1 − {self.h_l1}) × {r['l2_miss_penalty']:.4f}")

print(f"    = {self.t_l1} + {(1-self.h_l1)*r['l2_miss_penalty']:.4f}")

print(f"    ≈ {r['amat_cycles']:.3f} cycles  ≈ {r['amat_ns']:.2f} ns  (at 500 MHz)")

print("\n  AMAT Sensitivity (varying L1 hit rate):")

print(f"  {'L1 Hit Rate':>12} {'AMAT (cycles)':>15} {'AMAT (ns)':>12} {'Note':>10}")

print("  " + "-"*50)

for row in self.sensitivity():

print(f"  {row['h_l1']*100:>11.0f}% {row['amat_cycles']:>15.3f} "

f"{row['amat_ns']:>12.2f} {row['note']:>10}")

# ─────────────────────────────────────────────────────────────

# 3.4  TLB Simulation (64-entry fully-associative LRU)

# ─────────────────────────────────────────────────────────────

class TLB:

"""

64-entry fully-associative TLB with LRU replacement.

Maps Virtual Page Number (VPN) → Physical Frame Number (PFN).

"""

PAGE_SIZE   = 4096          # 4 KB

ENTRIES     = 64

TLB_LATENCY = 1             # cycles

def __init__(self):

self.table: OrderedDict = OrderedDict()   # {vpn: pfn}

self.hits   = 0

self.misses = 0

# Pre-warm TLB with common sensor buffer pages

for vpn in range(0x200, 0x210):

pfn = vpn + 0x800

self.table[vpn] = pfn

def _vpn(self, va: int) -> int:

return va >> 12       # upper 20 bits

def _offset(self, va: int) -> int:

return va & 0xFFF     # lower 12 bits

def translate(self, va: int, page_table: Dict[int, int]) -> Tuple[int, int, bool]:

"""

Translate virtual address to physical address.

Returns (physical_addr, latency_cycles, tlb_hit).

"""

vpn = self._vpn(va)

if vpn in self.table:            # TLB HIT

pfn = self.table[vpn]

self.table.move_to_end(vpn)

pa = (pfn << 12) | self._offset(va)

self.hits += 1

return pa, self.TLB_LATENCY, True

# TLB MISS — page table walk (5–10 cycles if PT cached in L2)

self.misses += 1

pfn = page_table.get(vpn, vpn + 0x500)    # default mapping

if len(self.table) >= self.ENTRIES:

self.table.popitem(last=False)          # evict LRU

self.table[vpn] = pfn

pa = (pfn << 12) | self._offset(va)

return pa, 8, False              # TLB miss = 8 cycles (PT walk in L2)

@property

def hit_rate(self) -> float:

total = self.hits + self.misses

return self.hits / total if total else 0.0

class VirtualMemorySystem:

"""

32-bit virtual address space: 20-bit VPN + 12-bit offset.

Physical space: 32-bit (up to 4 GB).

Page table: 2^20 PTEs × 4 B = 4 MB.

"""

def __init__(self, amat_calc: AMATCalculator):

self.amat_calc   = amat_calc

self.tlb         = TLB()

self.page_table  = {}         # VPN → PFN mapping

self._init_page_table()

def _init_page_table(self):

"""Simulate a page table for the sensor buffer region."""

for vpn in range(0, 0x1000):

self.page_table[vpn] = vpn + 0x100    # simple offset mapping

def address_translation_trace(self, va: int):

"""

Demonstrate step-by-step address translation for a given VA.

"""

vpn    = va >> 12

offset = va & 0xFFF

pa, latency, hit = self.tlb.translate(va, self.page_table)

pfn = pa >> 12

print("\n" + "="*70)

print(f"  ADDRESS TRANSLATION TRACE  (VA = 0x{va:08X})")

print("="*70)

steps = [

("1. Extract VPN", f"VA[31:12] = VPN", f"VPN = 0x{vpn:05X}"),

("2. Extract Offset", f"VA[11:0]  = Offset", f"Offset = 0x{offset:03X}"),

("3. TLB Lookup", f"Search TLB for VPN = 0x{vpn:05X}",

f"{'TLB HIT' if hit else 'TLB MISS (page table walk)'}  → PFN = 0x{pfn:05X}"),

("4. Form PA", f"PA = PFN[31:12] | Offset",

f"PA = 0x{pa:08X}"),

("5. Cache Lookup", f"Use PA to index L1-D cache",

f"L1-D access → latency = {latency} (TLB) + 2 (L1) = {latency+2} cycles"),

]

for label, op, result in steps:

print(f"  {label:<20} : {op:<40} → {result}")

def emat(self, tlb_hit_rate=0.985) -> float:

"""

EMAT = h_TLB × (t_TLB + AMAT) + (1 − h_TLB) × (t_PTwalk + AMAT)

"""

amat_r = self.amat_calc.compute()

amat   = amat_r["amat_cycles"]

t_tlb  = TLB.TLB_LATENCY

t_pt   = 8        # page-table walk (L2-cached)

emat   = tlb_hit_rate * (t_tlb + amat) + (1 - tlb_hit_rate) * (t_pt + amat)

return emat

def emat_report(self, tlb_hit_rate=0.985):

amat   = self.amat_calc.compute()["amat_cycles"]

e      = self.emat(tlb_hit_rate)

t_tlb  = TLB.TLB_LATENCY

t_pt   = 8

print("\n" + "="*70)

print("  EFFECTIVE MEMORY ACCESS TIME WITH TLB (Section 3.3.3)")

print("="*70)

print(f"\n  TLB hit rate    = {tlb_hit_rate*100:.1f}%")

print(f"  TLB hit latency = {t_tlb} cycle")

print(f"  TLB miss penalty= {t_pt} cycles  (page-table walk, L2-cached)")

print(f"  AMAT            = {amat:.3f} cycles")

print(f"\n  EMAT = h_TLB × (t_TLB + AMAT) + (1−h_TLB) × (t_PT + AMAT)")

print(f"       = {tlb_hit_rate} × ({t_tlb} + {amat:.3f})")

print(f"         + {1-tlb_hit_rate} × ({t_pt} + {amat:.3f})")

term1 = tlb_hit_rate * (t_tlb + amat)

term2 = (1 - tlb_hit_rate) * (t_pt + amat)

print(f"       = {term1:.2f} + {term2:.2f}")

print(f"       = {e:.2f} cycles")

# ─────────────────────────────────────────────────────────────

# 3.5  Cache Hierarchy Simulation (access trace)

# ─────────────────────────────────────────────────────────────

def run_cache_simulation():

"""

Simulate sensor-loop memory accesses through the L1/L2/L3 hierarchy.

Access pattern: sequential sensor array traversal with high temporal locality.

"""

l1d = LRUCache(L1D)

l2  = LRUCache(L2)

l3  = LRUCache(L3)

# Simulate 1000 memory accesses: sensor array read, then repeated access

addresses = []

base = 0x2000

# First pass: sequential (cold misses)

for i in range(64):

addresses.append(base + i * 4)

# Second pass: same addresses (hot hits)

for i in range(64):

addresses.append(base + i * 4)

# Third pass: partial — 80% from hot set, 20% new

for i in range(64):

if i % 5 == 0:

addresses.append(base + 256 + i * 4)  # cold

else:

addresses.append(base + i * 4)        # hot

total_latency = 0

for addr in addresses:

val = addr & 0xFF

hit1, lat1 = l1d.access(addr, val)

if not hit1:

hit2, lat2 = l2.access(addr, val)

if not hit2:

hit3, lat3 = l3.access(addr, val)

if not hit3:

total_latency += DRAM_LATENCY

total_latency += lat3

total_latency += lat2

total_latency += lat1

print("\n" + "="*70)

print("  CACHE HIERARCHY SIMULATION RESULTS (Section 3.1)")

print("="*70)

print(f"\n  Total accesses  : {len(addresses)}")

print(f"  Total latency   : {total_latency} cycles")

print(f"  Avg per access  : {total_latency/len(addresses):.2f} cycles\n")

for cache in [l1d, l2, l3]:

print(f"  {cache.stats()}")

# ─────────────────────────────────────────────────────────────

# 3.6  Memory Technology Comparison

# ─────────────────────────────────────────────────────────────

def memory_technology_comparison():

technologies = [

{

"name":         "SRAM (6T)",

"read_lat":     "1–5 ns",

"write_lat":    "1–5 ns",

"endurance":    ">10¹⁵ cycles",

"bandwidth":    ">100 GB/s",

"nonvolatile":  "No",

"cost_gb":      "~$5,000",

"density":      "Low",

"standby_pwr":  "Low",

"imp1_role":    "L1, L2, L3 cache",

"maturity":     "Mature"

},

{

"name":         "DRAM (LPDDR4)",

"read_lat":     "40–60 ns",

"write_lat":    "40–60 ns",

"endurance":    ">10¹⁵ cycles",

"bandwidth":    "51.2 GB/s",

"nonvolatile":  "No",

"cost_gb":      "~$5",

"density":      "High",

"standby_pwr":  "Medium (refresh)",

"imp1_role":    "Main Memory (2 GB)",

"maturity":     "Mature"

},

{

"name":         "NAND Flash",

"read_lat":     "25–100 µs",

"write_lat":    "200–1500 µs",

"endurance":    "10³–10⁵ cycles",

"bandwidth":    "0.5–3 GB/s",

"nonvolatile":  "Yes",

"cost_gb":      "~$0.10",

"density":      "Very High",

"standby_pwr":  "Zero",

"imp1_role":    "Sensor event log (64 GB)",

"maturity":     "Mature"

},

{

"name":         "MRAM",

"read_lat":     "2–10 ns",

"write_lat":    "2–10 ns",

"endurance":    ">10¹² cycles",

"bandwidth":    ">10 GB/s",

"nonvolatile":  "Yes",

"cost_gb":      "~$50",

"density":      "Medium",

"standby_pwr":  "Zero",

"imp1_role":    "NV-cache (future, 8 MB)",

"maturity":     "Emerging production"

},

{

"name":         "RRAM",

"read_lat":     "10–50 ns",

"write_lat":    "10–50 ns",

"endurance":    "10⁶–10¹² cycles",

"bandwidth":    "1–10 GB/s",

"nonvolatile":  "Yes",

"cost_gb":      "~$10 (proj.)",

"density":      "High (emerging)",

"standby_pwr":  "Zero",

"imp1_role":    "Fast event buffer (future)",

"maturity":     "Early production"

},

]

print("\n" + "="*110)

print("  MEMORY TECHNOLOGY COMPARISON (Section 3.4)")

print("="*110)

cols = ("Technology", "Read Lat.", "NV?", "Endurance", "Cost/GB", "IMP-1 Role")

fmt  = "  {:<18} {:>12} {:>5} {:>16} {:>10} {}"

print(fmt.format(*cols))

print("  " + "-"*100)

for t in technologies:

print(fmt.format(

t["name"], t["read_lat"], t["nonvolatile"],

t["endurance"], t["cost_gb"], t["imp1_role"]

))

print("\n  Selection Rationale:")

print("  SRAM  : Only technology meeting 2–30 cycle cache latency at KB–MB scales.")

print("  LPDDR4: Lowest cost/GB; 0.6V core (50% power vs DDR4); 51.2 GB/s for DMA bursts.")

print("  NAND  : Non-volatile long-term sensor log; wear-leveling manages 10⁵ cycle limit.")

print("  MRAM  : Recommended future upgrade — near-SRAM speed + non-volatile (alarm state).")

print("  RRAM  : Too immature for safety-critical deployment — future roadmap item.")

# ─────────────────────────────────────────────────────────────

# Main

# ─────────────────────────────────────────────────────────────

if __name__ == "__main__":

print("\n  IMP-1 Industrial Monitoring Processor — Section 3")

print("  Computer Architecture CSA1203 | CO4\n")

# Print cache configuration

print("="*70)

print("  CACHE HIERARCHY CONFIGURATION")

print("="*70)

for cfg in [L1I, L1D, L2, L3]:

print(f"  {cfg}")

print(f"  DRAM: LPDDR4, 200-cycle latency, 51.2 GB/s")

# Run simulation

run_cache_simulation()

# AMAT calculation

amat_calc = AMATCalculator()

amat_calc.report()

# Virtual memory

vm = VirtualMemorySystem(amat_calc)

vm.address_translation_trace(0x00402010)    # sensor buffer VA from report

vm.emat_report(0.985)

# Memory technology comparison

memory_technology_comparison()

5)

"""

Section 3: Hierarchical Memory System & Performance Evaluation

CSA1203 – Computer Architecture | IMP-1 Industrial Monitoring Processor

CO Mapping: CO4 | Marks: 25

Implements:

- 3-level cache hierarchy simulation (L1-I, L1-D, L2, L3)

- LRU / Pseudo-LRU replacement policy

- AMAT calculation (step-by-step and sensitivity analysis)

- Virtual memory with paging (32-bit, 4KB pages)

- TLB simulation (64-entry fully-associative, LRU)

- EMAT calculation

- Memory technology comparison table

"""

import random

from collections import OrderedDict

from dataclasses import dataclass

from typing import Dict, List, Optional, Tuple

# ─────────────────────────────────────────────────────────────

# 3.1  Cache Configuration

# ─────────────────────────────────────────────────────────────

@dataclass

class CacheConfig:

name:          str

capacity_kb:   int

associativity: int      # ways

block_size_b:  int      # bytes per block (64)

hit_latency:   int      # cycles

hit_rate:      float    # fraction of accesses that hit THIS level

write_policy:  str      # "write-back" | "write-through" | "read-only"

replacement:   str      # "LRU" | "Pseudo-LRU"

power_mw:      int

@property

def num_sets(self) -> int:

return (self.capacity_kb * 1024) // (self.associativity * self.block_size_b)

@property

def miss_rate(self) -> float:

return 1.0 - self.hit_rate

def __str__(self):

return (f"{self.name}: {self.capacity_kb} KB, "

f"{self.associativity}-way SA, "

f"{self.num_sets} sets, "

f"{self.hit_latency}-cycle hit, "

f"{self.hit_rate*100:.0f}% hit-rate")

# IMP-1 cache hierarchy definitions

L1I = CacheConfig("L1-I", 32,  4,  64,  2,  0.97, "read-only",  "LRU",        45)

L1D = CacheConfig("L1-D", 32,  4,  64,  2,  0.92, "write-back", "LRU",        45)

L2  = CacheConfig("L2",   256, 8,  64,  8,  0.97, "write-back", "Pseudo-LRU", 120)

L3  = CacheConfig("L3",  2048, 16, 64, 30,  0.99, "write-back", "Pseudo-LRU", 400)

DRAM_LATENCY = 200   # cycles

# ─────────────────────────────────────────────────────────────

# 3.2  LRU Cache Simulator (set-associative)

# ─────────────────────────────────────────────────────────────

class LRUCache:

"""

General-purpose set-associative LRU cache.

Each set is an OrderedDict: {tag: data} (MRU at right end).

"""

def __init__(self, cfg: CacheConfig):

self.cfg      = cfg

self.sets: List[OrderedDict] = [OrderedDict() for _ in range(cfg.num_sets)]

self.hits     = 0

self.misses   = 0

self.evictions = 0

def _parse_addr(self, addr: int) -> Tuple[int, int, int]:

"""Split address into (tag, set_index, block_offset)."""

offset_bits = int(self.cfg.block_size_b).bit_length() - 1

index_bits  = int(self.cfg.num_sets).bit_length() - 1

offset      = addr & ((1 << offset_bits) - 1)

index       = (addr >> offset_bits) & ((1 << index_bits) - 1)

tag         = addr >> (offset_bits + index_bits)

return tag, index, offset

def access(self, addr: int, data: int = 0, write: bool = False) -> Tuple[bool, int]:

"""

Perform a cache access.

Returns (hit: bool, latency: int).

"""

tag, idx, _ = self._parse_addr(addr)

s = self.sets[idx]

if tag in s:                     # HIT

self.hits += 1

s.move_to_end(tag)           # update LRU order

return True, self.cfg.hit_latency

# MISS

self.misses += 1

if len(s) >= self.cfg.associativity:

s.popitem(last=False)        # evict LRU

self.evictions += 1

s[tag] = data                    # install block

return False, self.cfg.hit_latency

@property

def hit_rate_observed(self) -> float:

total = self.hits + self.misses

return self.hits / total if total else 0.0

def stats(self) -> str:

total = self.hits + self.misses

return (f"{self.cfg.name}: {total} accesses | "

f"{self.hits} hits ({self.hit_rate_observed*100:.1f}%) | "

f"{self.misses} misses | {self.evictions} evictions")

# ─────────────────────────────────────────────────────────────

# 3.3  AMAT Calculator

# ─────────────────────────────────────────────────────────────

class AMATCalculator:

"""

Computes Average Memory Access Time analytically using the

recursive inclusive formula:

AMAT = h1*t1 + (1-h1)*[h2*t2 + (1-h2)*[h3*t3 + (1-h3)*t_DRAM]]

"""

def __init__(self,

h_l1=0.92, t_l1=2,

h_l2=0.97, t_l2=8,

h_l3=0.99, t_l3=30,

t_dram=200):

self.h_l1, self.t_l1 = h_l1, t_l1

self.h_l2, self.t_l2 = h_l2, t_l2

self.h_l3, self.t_l3 = h_l3, t_l3

self.t_dram          = t_dram

def compute(self) -> Dict:

# Step 1: L3 miss penalty

l3_miss = self.t_l3 + (1 - self.h_l3) * self.t_dram

# Step 2: L2 miss penalty (conditional miss → L3 or DRAM)

l2_miss = self.t_l2 + (1 - self.h_l2) * l3_miss

# Step 3: AMAT (from L1 perspective)

amat    = self.t_l1 + (1 - self.h_l1) * l2_miss

return {

"l3_miss_penalty": l3_miss,

"l2_miss_penalty": l2_miss,

"amat_cycles":     amat,

"amat_ns":         amat * 2,          # at 500 MHz, 1 cycle = 2 ns

}

def sensitivity(self) -> List[Dict]:

"""Vary L1 hit rate and report AMAT impact."""

rows = []

for h1 in [0.85, 0.90, 0.92, 0.95, 0.98]:

calc = AMATCalculator(h_l1=h1, t_l1=self.t_l1,

h_l2=self.h_l2, t_l2=self.t_l2,

h_l3=self.h_l3, t_l3=self.t_l3,

t_dram=self.t_dram)

r = calc.compute()

rows.append({

"h_l1": h1,

"amat_cycles": r["amat_cycles"],

"amat_ns":     r["amat_ns"],

"note": "baseline" if h1 == self.h_l1 else ""

})

return rows

def report(self):

r = self.compute()

print("\n" + "="*70)

print("  AMAT STEP-BY-STEP CALCULATION (Section 3.2)")

print("="*70)

print(f"\n  Given:")

print(f"    L1-D: hit-rate={self.h_l1*100:.0f}%  latency={self.t_l1} cycles")

print(f"    L2  : hit-rate={self.h_l2*100:.0f}%  latency={self.t_l2} cycles  (of L1 misses)")

print(f"    L3  : hit-rate={self.h_l3*100:.0f}%  latency={self.t_l3} cycles  (of L2 misses)")

print(f"    DRAM: latency={self.t_dram} cycles")

print(f"\n  Step 1 — L3 miss penalty:")

print(f"    = t_L3 + (1 − h_L3) × t_DRAM")

print(f"    = {self.t_l3} + (1 − {self.h_l3}) × {self.t_dram}")

print(f"    = {self.t_l3} + {(1-self.h_l3)*self.t_dram:.2f}")

print(f"    = {r['l3_miss_penalty']:.2f} cycles")

print(f"\n  Step 2 — L2 miss penalty:")

print(f"    = t_L2 + (1 − h_L2) × L3_miss_penalty")

print(f"    = {self.t_l2} + (1 − {self.h_l2}) × {r['l3_miss_penalty']:.2f}")

print(f"    = {self.t_l2} + {(1-self.h_l2)*r['l3_miss_penalty']:.4f}")

print(f"    = {r['l2_miss_penalty']:.4f} cycles")

print(f"\n  Step 3 — AMAT:")

print(f"    = t_L1 + (1 − h_L1) × L2_miss_penalty")

print(f"    = {self.t_l1} + (1 − {self.h_l1}) × {r['l2_miss_penalty']:.4f}")

print(f"    = {self.t_l1} + {(1-self.h_l1)*r['l2_miss_penalty']:.4f}")

print(f"    ≈ {r['amat_cycles']:.3f} cycles  ≈ {r['amat_ns']:.2f} ns  (at 500 MHz)")

print("\n  AMAT Sensitivity (varying L1 hit rate):")

print(f"  {'L1 Hit Rate':>12} {'AMAT (cycles)':>15} {'AMAT (ns)':>12} {'Note':>10}")

print("  " + "-"*50)

for row in self.sensitivity():

print(f"  {row['h_l1']*100:>11.0f}% {row['amat_cycles']:>15.3f} "

f"{row['amat_ns']:>12.2f} {row['note']:>10}")

# ─────────────────────────────────────────────────────────────

# 3.4  TLB Simulation (64-entry fully-associative LRU)

# ─────────────────────────────────────────────────────────────

class TLB:

"""

64-entry fully-associative TLB with LRU replacement.

Maps Virtual Page Number (VPN) → Physical Frame Number (PFN).

"""

PAGE_SIZE   = 4096          # 4 KB

ENTRIES     = 64

TLB_LATENCY = 1             # cycles

def __init__(self):

self.table: OrderedDict = OrderedDict()   # {vpn: pfn}

self.hits   = 0

self.misses = 0

# Pre-warm TLB with common sensor buffer pages

for vpn in range(0x200, 0x210):

pfn = vpn + 0x800

self.table[vpn] = pfn

def _vpn(self, va: int) -> int:

return va >> 12       # upper 20 bits

def _offset(self, va: int) -> int:

return va & 0xFFF     # lower 12 bits

def translate(self, va: int, page_table: Dict[int, int]) -> Tuple[int, int, bool]:

"""

Translate virtual address to physical address.

Returns (physical_addr, latency_cycles, tlb_hit).

"""

vpn = self._vpn(va)

if vpn in self.table:            # TLB HIT

pfn = self.table[vpn]

self.table.move_to_end(vpn)

pa = (pfn << 12) | self._offset(va)

self.hits += 1

return pa, self.TLB_LATENCY, True

# TLB MISS — page table walk (5–10 cycles if PT cached in L2)

self.misses += 1

pfn = page_table.get(vpn, vpn + 0x500)    # default mapping

if len(self.table) >= self.ENTRIES:

self.table.popitem(last=False)          # evict LRU

self.table[vpn] = pfn

pa = (pfn << 12) | self._offset(va)

return pa, 8, False              # TLB miss = 8 cycles (PT walk in L2)

@property

def hit_rate(self) -> float:

total = self.hits + self.misses

return self.hits / total if total else 0.0

class VirtualMemorySystem:

"""

32-bit virtual address space: 20-bit VPN + 12-bit offset.

Physical space: 32-bit (up to 4 GB).

Page table: 2^20 PTEs × 4 B = 4 MB.

"""

def __init__(self, amat_calc: AMATCalculator):

self.amat_calc   = amat_calc

self.tlb         = TLB()

self.page_table  = {}         # VPN → PFN mapping

self._init_page_table()

def _init_page_table(self):

"""Simulate a page table for the sensor buffer region."""

for vpn in range(0, 0x1000):

self.page_table[vpn] = vpn + 0x100    # simple offset mapping

def address_translation_trace(self, va: int):

"""

Demonstrate step-by-step address translation for a given VA.

"""

vpn    = va >> 12

offset = va & 0xFFF

pa, latency, hit = self.tlb.translate(va, self.page_table)

pfn = pa >> 12

print("\n" + "="*70)

print(f"  ADDRESS TRANSLATION TRACE  (VA = 0x{va:08X})")

print("="*70)

steps = [

("1. Extract VPN", f"VA[31:12] = VPN", f"VPN = 0x{vpn:05X}"),

("2. Extract Offset", f"VA[11:0]  = Offset", f"Offset = 0x{offset:03X}"),

("3. TLB Lookup", f"Search TLB for VPN = 0x{vpn:05X}",

f"{'TLB HIT' if hit else 'TLB MISS (page table walk)'}  → PFN = 0x{pfn:05X}"),

("4. Form PA", f"PA = PFN[31:12] | Offset",

f"PA = 0x{pa:08X}"),

("5. Cache Lookup", f"Use PA to index L1-D cache",

f"L1-D access → latency = {latency} (TLB) + 2 (L1) = {latency+2} cycles"),

]

for label, op, result in steps:

print(f"  {label:<20} : {op:<40} → {result}")

def emat(self, tlb_hit_rate=0.985) -> float:

"""

EMAT = h_TLB × (t_TLB + AMAT) + (1 − h_TLB) × (t_PTwalk + AMAT)

"""

amat_r = self.amat_calc.compute()

amat   = amat_r["amat_cycles"]

t_tlb  = TLB.TLB_LATENCY

t_pt   = 8        # page-table walk (L2-cached)

emat   = tlb_hit_rate * (t_tlb + amat) + (1 - tlb_hit_rate) * (t_pt + amat)

return emat

def emat_report(self, tlb_hit_rate=0.985):

amat   = self.amat_calc.compute()["amat_cycles"]

e      = self.emat(tlb_hit_rate)

t_tlb  = TLB.TLB_LATENCY

t_pt   = 8

print("\n" + "="*70)

print("  EFFECTIVE MEMORY ACCESS TIME WITH TLB (Section 3.3.3)")

print("="*70)

print(f"\n  TLB hit rate    = {tlb_hit_rate*100:.1f}%")

print(f"  TLB hit latency = {t_tlb} cycle")

print(f"  TLB miss penalty= {t_pt} cycles  (page-table walk, L2-cached)")

print(f"  AMAT            = {amat:.3f} cycles")

print(f"\n  EMAT = h_TLB × (t_TLB + AMAT) + (1−h_TLB) × (t_PT + AMAT)")

print(f"       = {tlb_hit_rate} × ({t_tlb} + {amat:.3f})")

print(f"         + {1-tlb_hit_rate} × ({t_pt} + {amat:.3f})")

term1 = tlb_hit_rate * (t_tlb + amat)

term2 = (1 - tlb_hit_rate) * (t_pt + amat)

print(f"       = {term1:.2f} + {term2:.2f}")

print(f"       = {e:.2f} cycles")

# ─────────────────────────────────────────────────────────────

# 3.5  Cache Hierarchy Simulation (access trace)

# ─────────────────────────────────────────────────────────────

def run_cache_simulation():

"""

Simulate sensor-loop memory accesses through the L1/L2/L3 hierarchy.

Access pattern: sequential sensor array traversal with high temporal locality.

"""

l1d = LRUCache(L1D)

l2  = LRUCache(L2)

l3  = LRUCache(L3)

# Simulate 1000 memory accesses: sensor array read, then repeated access

addresses = []

base = 0x2000

# First pass: sequential (cold misses)

for i in range(64):

addresses.append(base + i * 4)

# Second pass: same addresses (hot hits)

for i in range(64):

addresses.append(base + i * 4)

# Third pass: partial — 80% from hot set, 20% new

for i in range(64):

if i % 5 == 0:

addresses.append(base + 256 + i * 4)  # cold

else:

addresses.append(base + i * 4)        # hot

total_latency = 0

for addr in addresses:

val = addr & 0xFF

hit1, lat1 = l1d.access(addr, val)

if not hit1:

hit2, lat2 = l2.access(addr, val)

if not hit2:

hit3, lat3 = l3.access(addr, val)

if not hit3:

total_latency += DRAM_LATENCY

total_latency += lat3

total_latency += lat2

total_latency += lat1

print("\n" + "="*70)

print("  CACHE HIERARCHY SIMULATION RESULTS (Section 3.1)")

print("="*70)

print(f"\n  Total accesses  : {len(addresses)}")

print(f"  Total latency   : {total_latency} cycles")

print(f"  Avg per access  : {total_latency/len(addresses):.2f} cycles\n")

for cache in [l1d, l2, l3]:

print(f"  {cache.stats()}")

# ─────────────────────────────────────────────────────────────

# 3.6  Memory Technology Comparison

# ─────────────────────────────────────────────────────────────

def memory_technology_comparison():

technologies = [

{

"name":         "SRAM (6T)",

"read_lat":     "1–5 ns",

"write_lat":    "1–5 ns",

"endurance":    ">10¹⁵ cycles",

"bandwidth":    ">100 GB/s",

"nonvolatile":  "No",

"cost_gb":      "~$5,000",

"density":      "Low",

"standby_pwr":  "Low",

"imp1_role":    "L1, L2, L3 cache",

"maturity":     "Mature"

},

{

"name":         "DRAM (LPDDR4)",

"read_lat":     "40–60 ns",

"write_lat":    "40–60 ns",

"endurance":    ">10¹⁵ cycles",

"bandwidth":    "51.2 GB/s",

"nonvolatile":  "No",

"cost_gb":      "~$5",

"density":      "High",

"standby_pwr":  "Medium (refresh)",

"imp1_role":    "Main Memory (2 GB)",

"maturity":     "Mature"

},

{

"name":         "NAND Flash",

"read_lat":     "25–100 µs",

"write_lat":    "200–1500 µs",

"endurance":    "10³–10⁵ cycles",

"bandwidth":    "0.5–3 GB/s",

"nonvolatile":  "Yes",

"cost_gb":      "~$0.10",

"density":      "Very High",

"standby_pwr":  "Zero",

"imp1_role":    "Sensor event log (64 GB)",

"maturity":     "Mature"

},

{

"name":         "MRAM",

"read_lat":     "2–10 ns",

"write_lat":    "2–10 ns",

"endurance":    ">10¹² cycles",

"bandwidth":    ">10 GB/s",

"nonvolatile":  "Yes",

"cost_gb":      "~$50",

"density":      "Medium",

"standby_pwr":  "Zero",

"imp1_role":    "NV-cache (future, 8 MB)",

"maturity":     "Emerging production"

},

{

"name":         "RRAM",

"read_lat":     "10–50 ns",

"write_lat":    "10–50 ns",

"endurance":    "10⁶–10¹² cycles",

"bandwidth":    "1–10 GB/s",

"nonvolatile":  "Yes",

"cost_gb":      "~$10 (proj.)",

"density":      "High (emerging)",

"standby_pwr":  "Zero",

"imp1_role":    "Fast event buffer (future)",

"maturity":     "Early production"

},

]

print("\n" + "="*110)

print("  MEMORY TECHNOLOGY COMPARISON (Section 3.4)")

print("="*110)

cols = ("Technology", "Read Lat.", "NV?", "Endurance", "Cost/GB", "IMP-1 Role")

fmt  = "  {:<18} {:>12} {:>5} {:>16} {:>10} {}"

print(fmt.format(*cols))

print("  " + "-"*100)

for t in technologies:

print(fmt.format(

t["name"], t["read_lat"], t["nonvolatile"],

t["endurance"], t["cost_gb"], t["imp1_role"]

))

print("\n  Selection Rationale:")

print("  SRAM  : Only technology meeting 2–30 cycle cache latency at KB–MB scales.")

print("  LPDDR4: Lowest cost/GB; 0.6V core (50% power vs DDR4); 51.2 GB/s for DMA bursts.")

print("  NAND  : Non-volatile long-term sensor log; wear-leveling manages 10⁵ cycle limit.")

print("  MRAM  : Recommended future upgrade — near-SRAM speed + non-volatile (alarm state).")

print("  RRAM  : Too immature for safety-critical deployment — future roadmap item.")

# ─────────────────────────────────────────────────────────────

# Main

# ─────────────────────────────────────────────────────────────

if __name__ == "__main__":

print("\n  IMP-1 Industrial Monitoring Processor — Section 3")

print("  Computer Architecture CSA1203 | CO4\n")

# Print cache configuration

print("="*70)

print("  CACHE HIERARCHY CONFIGURATION")

print("="*70)

for cfg in [L1I, L1D, L2, L3]:

print(f"  {cfg}")

print(f"  DRAM: LPDDR4, 200-cycle latency, 51.2 GB/s")

# Run simulation

run_cache_simulation()

# AMAT calculation

amat_calc = AMATCalculator()

amat_calc.report()

# Virtual memory

vm = VirtualMemorySystem(amat_calc)

vm.address_translation_trace(0x00402010)    # sensor buffer VA from report

vm.emat_report(0.985)

# Memory technology comparison

memory_technology_comparison()